SA WG2 Meeting #149e	S2-2200594

# February 14th – 25th, 2022; Elbonia

Source: 	OPPO

Title: 	New KI for enhancements on QoS Sustainability analytics

Document for: 	Approval

Agenda Item: 	9.23

Work Item / Release:	eNA_Ph3 / Rel-18

Abstract of the contribution: The PCR is to propose KI for WT#3.8 regarding the possible enhancements on QoS Sustainability analytics.

1	Discussion

This key issue is applicable to WT#3.8 for the enhancements on QoS Sustainability analytics.

WT#3.8: enhancements on QoS Sustainability analytics.

- Whether and how to enhance the QoS sustainability analytics with finer granularity and additional input data.

As described in [x], The consumer of QoS Sustainability analytics may request the NWDAF analytics information regarding the QoS change statistics for an Analytics target period in the past in a certain area or the likelihood of a QoS change for an Analytics target period in the future in a certain area. However, the current QoS Sustainability analytics can only reflect whether the Reporting Threshold has been/ will be met during the Analytics target period in a certain area. For some specific cases, a finer granularity of QoS sustainability may be needed, such that the corresponding policy could be adjusted timely to keep user experience from being interrupted.

As the specific cases motioned above, the intent of this PCR is to study what would be the possible enhancements on QoS Sustainability analytics to support effective policy adjustments.

2	Proposal

The proposed KI is applicable to WT#3.8. It is proposed to update TR 23.700-81 on eNA_Ph3 as follows.

Start of Changes (all new text)

# 2	References

[x]	3GPP TS 23.288: " Architecture enhancements for 5G System (5GS) to support network data analytics services".

5	Key Issues

5.X	Key Issue #X: QoS Sustainability analytics enhancement on finer granularity area and additional input data

5.X.1	Description

In order to provide analytics information on QoS value per time slot per waypoint to some specific cases, it may be good to enhance the QoS sustainability analytics with finer granularity, additional input and output. Hence, the following aspects are expected to be studied for the possible the enhancements on QoS Sustainability analytics:

- Whether and how to define the use cases which are applicable to the QoS sustainability analytics with finer granularity, and how to define the finer granularity of QoS sustainability for each use case.

- What additional input data is required to derive such QoS sustainability analytics with finer granularity.

- What would be the additional output corresponding to such QoS sustainability analytics with finer granularity.

End of Changes