3GPP TSG-WG SA2 Meeting #149E e-meeting  	S2-2200595

# Elbonia, Feburary 14 – 25, 2022	(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0087 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Discussion and proposals for MBS SAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-01-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | LS from RAN2 (S2-2107037) asked the feasibility of introducing SAI in MBS in Rel-17. And in SA2#147 meeting, an LS (S2-2108175) was provided including a positive feedback to RAN2 but the details would be FFS. In SA2#148 meeting, the intial content was introduced for MBMS SAI-comparative definition for MBS, i.e., the followings are agreed: - Using SAI (or alike) in RAN SIB message to save the signalling cost in the SIB. - The mapping between SAI and frequency was configured at NG-RAN nodes via OAM. - AF provides UE the MBS XXX ID(s) of a broadcast MBS session and it would be used by the UE for frequency selection. While the following issues are to be further discussed/confirmed: - Terminology – MBS XXX ID is only for temporary usage. - The relationship between MBS XXX ID and MBS service area; - How could the AF get the MBS XXX ID; - How could the NG-RAN nodes know the MBS XXX ID for a certain broadcast MBS session. In general, we have the following considerations: - MBS XXX ID could be MBS frequency area ID, but the details can be determined during the meeting; - MB-SMF provides the information of SAI when the AF requests to create the (broadcast) sessions. - The RAN node may additionally get the information about the SAI for a certain TMGI from MB-SMF during broadcast MBS session start procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Complete the part for MBS XXX ID in clause 6.5.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature of the MBMS SAI-comparative ID in 5G MBS is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.5.4	MBS XXX ID

Editor's note:	The name MBS XXX ID is temporary and FFS.

The MBS XXX ID is used for broadcast MBS session to guide the frequency selection of the UE. MBS XXX ID identifies a preconfigured NG-RAN nodes within which the cell(s) announces the MBS XXX ID and the associating frequency (details see TS 38.300 [9]). When the UE knows that the MBS XXX ID is used for a certain TMGI by e.g., AF notification or pre-configuration, it can derive the frequency used for the TMGI by listening to the announcement from the cell.

A cell shall be able to belong to one or more MBS XXX, and therefore is associated with one or more MBS XXX IDs. Every MBS XXX ID shall be unique within a PLMN.

NOTE 1:	The MBS XXX ID is defined by the operator, and configured in the NG-RAN and MB-SMF via O&M.

NOTE 2:	The cells which do not belong to the preconfigured area may also announce the relationship between MBS XXX ID and frequency, via e.g., OAM.

NOTE 3:	An MBS XXX ID in a SIB does not guarantee that the broadcast service is available in a cell at a given time.

MBS XXX ID identifies a preconfigured area within which the cell(s) announces the MBS XXX ID and the associating frequency (details see TS 38.300 [9]). MBS XXX ID and their mapping to frequencies are provided to RAN nodes via OAM.

Based on this configuration, RAN nodes announce in SIBs over the radio interface information about the MBS XXX IDs and frequencies.

The MBS XXX ID(s) might be provided by MB-SMF to AF during MBS session Creation procedure in Clause 7.1.1.2 and 7.1.1.3. The MB-SMF determines the MBS XXX ID(s) corresponding to the MBS service, based on e.g., MBS service area, SLA and so on.

If MBS XXX ID(s) is provided by MB-SMF, the MBS XXX ID(s) of a broadcast MBS session are communicated in the service announcement towards the UE (as defined in Clause 6.11). The UE compares those MBS XXX IDs(s) with the MBS XXX ID(s) in SIBs for frequency selection.

During MBS Session Start for Broadcast defined in Clause 7.3.1, the MB-SMF may include the MBS XXX ID(s) in N2 message to the NG-RAN nodes. If MBS XXX ID(s) are included in the N2 message, the NG-RAN node uses the MBS XXX ID in the transmission of MBS data. Details see TS 38.300 [9] and TS 38.413 [15].

# * * * * End of changes * * * *