3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200607

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, CBN

Title:	Key Issue: 5MBS MOCN network sharing

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract: This document proposes the key issue on 5MBS MOCN network sharing.

1. Introduction

In SP#94e, new SID on 5G multicast-broadcast services phase 2 has been agreed [1] with the following work task:

WT#1.2 Study feasible and efficient resource utilization for the same broadcast content to be provided to 5G MOCN network sharing scenarios (i.e., multiple CNs are connected to the same NG-RAN); NOTE 2: The NG-RAN is assumed not to be aware of the same content via the application layer information detection. NOTE 3: Collaboration with RAN WGs is needed.

The enablers for 5MBS MOCN network sharing are mainly from the Mobile Operator Networks, such as Chinese operators CMCC and CBN, as it can bring resource efficiency and reduce CAPEX. In addition, such feature has been discussed and well-accepted by the RAN WGs. As per the approved WID NR_MBS_enh for Rel-18 [2]:

Network sharing is a common practice to reduce network CAPEX. With RAN sharing deployment, if the same Multicast/Broadcast service is provided by two (or more) operators separately, this service would be recognized as separate TMGIs resulting in duplicated PTM radio resources consumption in the same cell for transmission of the same content. This justifies resource efficiency improvement in the RAN sharing scenario.

Study and if necessary, specify enhancements to improve the resource efficiency for MBS reception in RAN sharing scenarios [RAN3]

Rel-18 MBS phase 2 work is envisioned to address the use cases of MOCN network sharing for 5MBS mentioned above. Rel-18 MBS should be able to harvest the benefits of MOCN network deployment in a way that does not require the shared RAN node broadcast the same service data from different PLMN of the MOCN more than once.

It is also important to consider the following issues to meet the requirement as described in WT#1.2:

-	How to utilize the potential enhancements on top of Release-17 5MBS mechanism to assist NG-RAN node to determine the same broadcast MBS content from different MNOs;

- Which entity (e.g., AF or NFs) could provide the assistance information to the shared NG-RAN?

-	Whether the UE need to aware the correlation of the same broadcast content from different MNOs.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47.

# * * * * First change * * * *

5	Key Issues

5.B	Key Issue #B: 5MBS MOCN network sharing

5.B.1	Description

To enable broadcast MBS content transmit via the 5G MOCN and achieve the efficient resource utilization,

The following aspects will be studied for broadcast:

-	Whether and how to enhance the Release-17 5MBS network functions and procedures to enable the shared NG-RAN nodes determine the broadcast MBS data provided by different MNOs is the same.

# * * * * End of changes * * * *

4. References

[1] SP-211324, New SID: Architectural enhancements for 5G multicast-broadcast services Phase 2.

[2] RP-213568, New WID: Enhancements of NR Multicast and Broadcast Services.