3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200647

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, Samsung (Rapporteur)

Title:	FS_GMEC TR 23.700-74 Scope

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: This paper introduces the TR 23.700-74 Scope

1. Introduction/Discussion

The scope for FS_GMEC TR 23.700-74 “Study on generic group management, exposure and communication enhancements”

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-74 V0.0.0.

# * * * * First change * * * *

1	Scope

The scope of this Technical Report is to study potential enhancements on generic group management, exposure and communication. Specifically, the following aspects are in the scope of the study:

-	Enhance group attribute management and group status event reporting:

-	 set/modify the group attributes: provisioning of service area or QoS applicable to each UE of a given group

-	 subscribe to group status event reporting for the event "newly registered or (de)-registered group member"

-	Study whether and how to enhance NEF exposure framework to enable capability exposure for  provisioning of traffic characteristics and monitoring of performance characteristics applicable to each UE of a given group

-	Support group communication for a 5G VN, which supports multiple SMFs, including support of SMF redundancy for reliability of the 5G VN group communication

-	Whether additional mechanism or enhancement is needed and how to support group communication allowing UE to simultaneously send data to different groups, where each group has a different QoS policy (requirement regarding 5SEI as indicated in clause 6.13.2 of TS 22.261 [6])

# * * * * Second change * * * *

# * * * * Third change * * * *

# * * * * Fourth change * * * *

# * * * * Fifth change * * * *

# * * * * End of changes * * * *