3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200672

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Samsung

Title:	New solution to Key Issue on Support for 5G Smart Energy and Infrastructure

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: This proposes a solution to Key Issue on Support for 5G Smart Energy and Infrastructure.

1. Introduction/Discussion

In Rel-18 SA1 study on 5G Smart Energy and Infrastructure “5SEI” has concluded new requirement (clause 6.13.2, 6.28 of TS 22.261 and clause 5.2, 5.6 and 9 of TS 22.104) including that the 5G system shall allow a UE to request a communication service to send data to different groups of UEs at the same time and the 5G system shall allow different QoS policy for each group the UE communicates with.

It is proposed to provide a new solution for this key issue.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-74 V0.0.0.

# * * * * First change * * * *

6.X	Solution #X: A PDU session with multiple groups

6.X.1	Introduction

This solution is Key Issue #X on 5G Smart Energy and Infrastructure.

Figure 6.X.1-1 depicts the concept of the solution.

Figure 6.X.1-1: Group communication enhancements for SEI (Smart Energy and Infrastructure)

6.X.2	Functional Description

Figure 6.X.2-1 depicts the concept of the solution.

Figure 6.X.2-1: A PDU session with multiple groups

The principles of this solution are like the following. The numbers of each bullet refer to those in Figure 6.X.2-1.

- A UE or an AF requests 5GS to configure new groups with different QoS’s (1).

- A UE establish a PDU Session for the groups with explicit priorities or implicit priorities (e.g. the former of the list means the higher priority) (1).

- The SMF sets up the UPF forwarding table based on the groups and the SMF sets up the QoS flows based on the QoS requirements (2, 3, 4).

- The UE sends the traffic to the first UPF with the highest QoS of the groups. The UPF forward the traffic to the UEs of a group with the QoS requirements of the group and to the UEs of another group with the dedicated QoS for the group. (5)

6.X.3	Procedures

Editor's Note: This clause describes high-level procedures and information flows for the solution.

6.X.4	Impacts on existing entities and interfaces

Editor's Note: This clause lists impacts to existing entities and interfaces.

# * * * * End of changes * * * *