SA WG2 Meeting #149e	S2-2200676

# February 14th – 25th, 2022; Elbonia               		(revision of S2-220)

Source: 	Samsung

Title: 	New KI related with FS_5TRS_URLLC WT1.2

Document for: 	Approval

Agenda Item: 	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract of the contribution: This proposes a Key Issue for FS_5TRS_URLLC based on the Work Task 1.2.

1	Discussion

The FS_5TRS_URLLC SID justification read

1) SA1 is also specifying requirements for 5G System to remain resilient if there is GNSS failure and for 5G System to act as a backup and offer wireless and indoor-capable time synchronization service for other applications (e.g. financial, power grid systems).

2) Generic TSC and exposure enhancements to 5GS for IP and ETH applications are needed for the following reasons:

- Current Exposure framework enables AF to request QoS parameters, provide traffic characteristics but not reliability aspect which is important for some IP and ETH applications

3) In order to support low latency and low jitter, it needs to be studied how to interwork with TSN Transport networks. It may be possible to apply TSN networks in the transport network. In such deployments, it may be beneficial to be able to interwork so that the transport network can also provide delay guarantees.

It has been raised whether RAN feedback on when the burst should be sent may be helpful. The performance benefit of such proposals can be studied and the gain can be compared with the complexity of such solutions.

The FS_5TRS_URLLC SID objectives read

WT#1. Support for 5G Timing Resiliency requirements defined by SA1.

WT#1.1. Study how to report 5GS network timing synchronization status (such as divergence from UTC and 5GS network timing source degradation) to UEs and 3rd party applications (AFs):

- Study how RAN and 5GC learn about network 5GS network timing synchronization status to be able to inform UEs and AFs.

- Study if additional information needs to be provided to UEs and AFs to inform about 5GS network timing synchronization status.

WT#1.2. Study how to enable AFs to request time synchronization service in a specific coverage area and how to enforce the coverage area.

WT#1.3. Study how to control 5G time synchronization service based on subscription (i.e. introducing subscription parameter for time synchronization and enforcing it).

NOTE: 	Configuration and operation of the synchronization network and mitigation actions when time source fails or degrades is assumed to be outside the scope of 3GPP, since there are protocols, techniques and practices already specified by other standardization bodies.

WT#2 Study how to enable an AF to explicitly provide PER to NEF/PCF.

WT#3. Support for low latency:

WT#3.1 Study mechanisms for interworking with TSN transport networks. Study interworking mechanisms with TSN networks deployed in the transport network in order to support of E2E determinism and low latency communication and efficient N3 transmission;

NOTE: 	WT#3.1 assumes that connected mode mobility is not used in the deployment for the corresponding traffic flows.

WT#3.2 Study if there is a need for applications to adapt downstream scheduling in order for 5GS to meet really low latency (e.g. 2msecs) requirement and if there is a need to have feedback from RAN (e.g. for application to consider DL packet transmission time slots to avoid buffering in the RAN) for this purpose.

2 Proposal

It is proposed to update TR 23.700-25 on FS_5TRS_URLLC as follows.

5	Key Issues

5.X	Key Issue #X: Time synchronization service enhancement to support coverage area

5.X.1	Description

This key issue is targeting how to support time synchronization service with coverage area.

For this key issue, the following areas should be studied:

- How to enable AFs to request time synchronization service in a specific coverage area.

- How to enforce the coverage for time synchronization service.