SA WG2 Meeting #149e	S2-2200722

# February 14th – 25th, 2022; Elbonia               	       (revision of S2-220xxxx)

Source: 	vivo

Title: 	FS_eNPN_Ph2 KI proposal - WT2

Document for: 	Approval

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This contribution proposes to add KI for supporting non-3GPP access for SNPN.

1	Discussion

The FS_eNPN_Ph2 SID justification reads

According to TR 23.700-07, there are yet two key issues not addressed in Rel-17 time frame, namely:

-	Support for equivalent SNPNs

The FS_eNPN_Ph2 SID objectives reads

1. Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection.

2 Proposal

It is proposed to update TR 23.700-08 on FS_eNPN_Ph2 as follows

5	Key Issues

5.X	Key Issue #X: Support for Non-3GPP Access for SNPN

5.X.1	Description

From Rel-16 a UE may access SNPN services via PLMN using N3IWF as described in TS 23.501 clause 5.30.2.8. From the SNPN point of view, the UE is accessing the SNPN using non-3GPP access. However, a UE using real non-3GPP access (e.g. WiFi, Bluetooth, Wireline) to access an SNPN is not supported.

This key issue aims at addressing the support for non-3GPP access for SNPN with the following considerations:

-  An SNPN may support a UE accessing via trusted non-3GPP access, untrusted non-3GPP access, or wireline access.

-  An SNPN may support a UE accessing via both a 3GPP access and a non-3GPP access.

-  Study whether the solution for support of non-3GPP access for PLMN can be reused, whether and how any enhancements are needed.

-  Study whether and how WLAN selection mechanism needs to be enhanced.

NOTE: The solution for support of non-3GPP for PLMN is expected to be reused as much as possible.

NOTE: Enhancements of a UE accessing to SNPN services via PLMN as described in TS 23.501 5.30.2.8 is not within the scope of this Key Issue.

NOTE: The UE simultaneously connects to an SNPN via a non-3GPP access and via PLMN using N3IWF are not within the scope of this Key Issue.