SA WG2 Meeting #149e	S2-2200796

# February 14th – 25th, 2022; Elbonia               			(revision of S2-220)

Source: 	Lenovo, Motorola Mobility

Title: 	Solution on WT#3 based on TNGF relocation

Document for: 	Approval

Agenda Item: 	9.13

Work Item / Release:	FS_5WWC_Ph2 / Rel-18

Abstract of the contribution:

# 1	Discussion

Presently, when the UE attempts to register with 5GC via trusted non-3GPP access, a TNGF is selected to serve this UE but without considering slice information. Consequently, the TNGF selected to serve the UE may not support the slices requested by the UE. To address this issue, a new solution is needed which can select a TNGF to serve the UE by using also slice information.

A TNGF can be selected based on slice information by using either:

- A UE-based solution, where the requested NSSAI provided by the UE is used to select an appropriate TNGF to serve the UE; or

- A network-based solution, where the AMF determines an appropriate TNGF to serve a UE based on the requested NSSAI, subscribed NSSAI, S-NSSAIs supported by available TNGFs, etc.

Both approaches a) and b) have pros and cons and need to be compared and analyzed in detail. However, this is not the purpose of this document. It is expected that such comparison and analysis will be conducted in future documents. The purpose of this document is to propose a network-based solution that adopts the approach b).

# 2 Proposal

It is proposed to insert the following solution in TR 23.700-17 as a candidate solution for the KI on “how to select a TNGF/N3IWF that supports the S-NSSAI(s) needed by the UE”.

FIRST CHANGE

# 6	Solutions

## 6.X	Solution #X: Registration via Trusted Non-3GPP Access with TNGF Relocation

### 6.X.1	Description

The solution in this clause defines extensions to the "5GC registration via trusted non-3GPP access" procedure, which enable the AMF to select a different TNGF to serve the UE, when the originally selected TNGF cannot support the slices needed by the UE.

The key steps of the solution are summarized as follows:

a)	The UE selects a PLMN and a trusted non-3GPP access that supports 5G connectivity with this PLMN, as specified in TS 23.501 clause 6.3.12.

b)	An EAP procedure is initiated between the UE and the selected trusted non-3GPP access. The realm provided by the UE in NAI ("<any_username>@nai.5gc.mnc<MNC>.mcc<MCC>.3gppnetwork.org") is used to select a TNGF that supports 5G connectivity with the selected PLMN. This TNGF is selected without considering any slice information.

c)	The UE sends a Registration Request message via the selected TNGF, as specified in TS 23.502 clause 4.12a.2.2. The selected TNGF selects an AMF and forwards to this AMF the Registration Request message. If the Requested NSSAI is received by the selected TNGF (according to the conditions defined in clause 5.15.9 of TS 23.501 [2]), then the TNGF selects an AMF by considering also the Requested NSSAI (as defined in clause 6.3.5 of TS 23.501 [2]).

d)	The AMF identifies whether the selected TNGF can or cannot support the slices requested by the UE. If the selected TNGF cannot support the slices requested by the UE, and if there is another TNGF that can serve this UE and can support the slices requested by the UE, then the AMF triggers TNGF relocation, i.e., requests to substitute the selected TNGF with a new TNGF. This TNGF relocation does not interrupt the ongoing registration procedure. The registration procedure continues but the UE is instructed to establish a NWt connection with the new TNGF.

The AMF can decide to trigger TNGF relocation based on its own criteria. Typically, the AMF considers the Requested NSSAI provided by the UE, the Subscribed NSSAI and the S-NSSAIs supported by all TNGFs that can serve the UE.

Editor’s note: Many TNGFs can be connected to the AMF. How the AMF can determine which of these TNGFs can serve the UE is FFS.

e)	As a result, the 5G registration procedure is completed and a new TNGF is selected by AMF, which can support all (or the most) S-NSSAIs requested by the UE.

More details can be found in the next clause.

### 6.X.2	Procedures

#### 6.X.2.1	Registration via trusted non-3GPP access with TNGF relocation

The procedure specified in this clause extends the registration procedure for trusted non-3GPP access specified in TS 23.502 [X], clause 4.12a.2.2, in order to support TNGF relocation. During the registration via trusted non-3GPP access, the AMF may trigger TNGF relocation as defined in the previous clause.

Figure 6.X.2.1-1: Registration via trusted non-3GPP access with TNGF relocation

The steps in Figure 6.X.2.1-1 are the same steps as in TS 23.502 [X], Figure 4.12a.2.2-1 (registration via trusted non-3GPP access) with the following additions and clarifications:

3.	The TNAP selects a TNGF, referred to as TNGF-1. The TNGF-1 is selected by using the NAI provided by the UE.

10a. The AMF decides to trigger TNGF relocation and use TNGF-2 instead of TNGF-1. For taking this decision, the AMF may consider the Requested NSSAI provided by the UE, the Subscribed NSSAI for this UE, the S-NSSAIs supported by all TNGFs that can serve the UE, and other information.

If the AMF decides to trigger TNGF relocation, then the AMF shall include in the N2 Initial Ctx Setup Request the TNGF-2 IP address, indicating that the AMF wants to relocate the UE to TNGF-2. When TNGF-1 receives message 10a, it recognizes that the TNGF Address contains an IP address of another TNGF, hence, it recognizes that the AMF requests TNGF relocation.

Editor’s note: Instead of sending an N2 Initial Ctx Setup Request in step 10a, the AMF could send a new N2 message used for TNGF relocation, e.g., a TNGF Relocation Command. In this case, the TNGF-1 would respond with a TNGF Relocation Response in step A1. It is FFS whether such a new N2 message is preferable.

10b. The TNGF Address sent to UE is the TNGF-2 IP address provided by AMF.

A1. After receiving an EAP-Response/5G-Notification packet from the UE in step 10c, the TNGF-1 shall send message 10d containing the EAP-Success packet. In addition, TNGF-1 shall respond to AMF with an N2 Initial Context Setup Failure message (see TS 38.413) with a Cause value indicating that the failure is because of the requested TNGF relocation. If needed, this message may also contain some information that shall be transparently forwarded to TNGF-2 (via the AMF).

A2. The N2 Initial Context Setup Failure message triggers the AMF to send an N2 Initial Context Setup Request to TNGF-2 including the TNGF key, the UE identity and, if received in step A1, the information that shall be transparently forwarded to TNGF-2.

13. The UE sets up a secure NWt connection using the TNGF address received in step 10b, i.e., with TNGF-2. All further communication between the UE and the AMF takes place via TNGF-2.

Note that, in the above registration procedure, the EAP-5G protocol runs between the UE and TNGF-1, while the NWt connection is established between the UE and TNGF-2.

### 6.X.3	Impacts on Existing Nodes and Functionality

UE:

-	No impact.

TNGF-1 (used for EAP-5G):

-	Shall be able to recognize when the AMF requests relocation (by inspecting the TNGF Address in step 10a) and shall respond to AMF with N2 Initial Context Setup Failure after receiving an EAP-Response/5G-Notification packet from the UE (which indicates that the UE has received the IP address of TNGF-2).

TNGF-2 (used for NWt connection establishment):

-	Shall be able to receive N2 Initial Context Setup Request from an AMF without having an EAP-5G session with a UE and shall be able to establish an NWt connection with the UE by using the UE identity and the TNGF key included in the N2 Initial Context Setup Request.

AMF:

-	Shall be able to trigger TNGF relocation, e.g., based on the Requested NSSAI provided by the UE, the Subscribed NSSAI for this UE, the S-NSSAIs supported by all TNGFs that can serve the UE, and other information. The AMF shall include in the N2 Initial Ctx Setup Request (step 10a) the IP address of the new TNGF.

-	After receiving the N2 Initial Context Setup Failure from TNGF-1 with a cause value indicating TNGF relocation, the AMF shall send and N2 Initial Context Setup Request to TNGF-2 including the TNGF key and the UE identity.

END OF CHANGES