3GPP TSG-WG SA2 Meeting #149E e-meeting  	S2-2200853

Elbonia, February 14 – 25, 2022

Title:	[DRAFT] LS on handover triggered by Target NSSAI

Response to:

Release:	Rel-17

Work Item:	eNS_Ph2

Source:                 SA2

To:                         RAN2, RAN3

Cc:

Contact Person:

Name:	Susan

E-mail Address:	Susan DOT Shishufeng AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2200851 (23.501 CR 3540), S2-2200852 (23.502 CR 3382)

1. Overall Description:

When the UE requests a new S-NSSAI not available in the current UE's Tracking Area in Registration Request, the AMF may provide a Target NSSAI and a corresponding RFSP index to the NG-RAN for the purpose of redirecting the UE to a cell in another TA supporting network slices not available in the current TA. The redirection mechanisms upon RRC release help the UE reselect a proper cell in another TA supporting the Target NSSAI. However, the active PDU Sessions will be deactivated when the UE enters into RRC Idle state during redirection.

If the NG-RAN is able to handover the UE to a nearby cell supporting the Target NSSAI, the active PDU Sessions of the UE will be unaffected.

In order to avoid the interruption to the active PDU Sessions of the UE, if there is any, the AMF provides a handover request indication with the Target NSSAI to the NG-RAN. SA2 asks kindly RAN2 and RAN3 to consider the following requirements to NG-RAN in Rel-17:

- When receiving the Target NSSAI together with a handover request indication from the AMF, the NG-RAN performs handover of the UE with the active PDU Sessions to a proper cell in another TA supporting the Target NSSAI, if available.

- The NG-RAN keeps the UE with the active PDU Sessions in the current TA if there is no proper cell nearby which can meet the criterion of the handover.

- Implement the changes on NGAP for the handover request indication.

In addition, SA2 kindly asks RAN2 and RAN3 whether the RFSP index corresponding to the Target NSSAI is useful for the NG-RAN to determine measurement configuration before preparing handover.

2. Actions:

To: RAN2 and RAN3

ACTION: 	SA2 kindly asks RAN2 and RAN3 to take the above information into consideration and provides answers to the RFSP index related question.

3. Date of Next TSG SA WG2 Meetings:

See https://portal.3gpp.org/Home.aspx?tbid=385&SubTB=385#/