SA WG2 Meeting #S2-149E	S2-2200861

# February 14th – 25th, 2022; Electronic meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3542 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  | UICC apps |  |  |  |  | ME | X | Radio Access Network |  |  |  |  |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Applicability of lists provided by VPLMN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | MINT |  |  |  |  |  |  |  |  |  | Date: |  |  | 2022-01-28 |  |  |  |
| Category: | F |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-17 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |  |  |  |
| Reason for change: |  | In C1-220849, CT1 has agreed for below indication: 'Applicability of "lists of PLMN(s) to be used in disaster condition" provided by a VPLMN' indication.This will be provided to the UE as part of UPU procedure by HPLMN.The UE uses this flag to determine if it has to use the list configured by VPLMN in roaming area for PLMN selection procedure as defined in TS 23.122 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align with CT1 agreement by including the indication in UE configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CT1 agreement not captured |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.40.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | x | Other core specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

***** Start of changes *****

### 5.40.2	UE configuration and provisioning for Disaster Roaming

A UE supporting Disaster Roaming is configured with the following information:

-	Activation of Disaster Roaming;

-	Optionally, 'Applicability of "lists of PLMN(s) to be used in disaster condition" provided by a VPLMN' indication;

-	Optionally, list of PLMN(s) to be used in Disaster Condition.

The Activation of Disaster Roaming is performed by the HPLMN using the UE Parameters Update Procedure as defined in TS 23.502 [3]. The UE shall not perform disaster roaming if HPLMN has not configured the UE with Activation of Disaster Roaming.

The optional 'list of PLMN(s) to be used in Disaster Condition' may be pre-configured in USIM or provided by the HPLMN during and after a successful registration procedure over 3GPP access or non-3GPP access via Registration Request procedure or UE Configuration Update procedure as defined in TS 23.502 [3]. The 'list of PLMN(s) to be used in Disaster Condition' may be configured over non-3GPP access before disaster condition has occurred.

While roaming (i.e. not in the country of the HPLMN), the Registered PLMN may provide the 'list of PLMN(s) to be used in Disaster Condition' during and after a successful registration procedure to the UE via Registration Request procedure or UE Configuration Update procedure as specified in TS 23.502 [3]. This list shall not alter any list provided by the HPLMN.

The details of the UE behaviour regarding the usage of this list and 'Applicability of "lists of PLMN(s) to be used in disaster condition" provided by a VPLMN' indication are described in TS 23.122 [17] and TS 24.501 [47].

If the UE is not configured with 'list of PLMN(s) to be used in Disaster Condition' follows the procedure described in TS 23.122 [17] to select PLMN to be used in Disaster Condition.

***** End of changes *****