3GPP TSG-WG SA2 Meeting #149E e-meeting  	S2-2200936

Elbonia, February 14 – 25, 2022	(revision of S2-220xxxx)

Title:	[DRAFT] Reply LS on Use, if any, of network provided "Indication of country of UE location"

Response to:	LS (S2-2200043/ C1-217150) on Use, if any, of network provided "Indication of country of UE location" from CT1

Release:	Release 17

Work Item:	5GSAT_ARCH

Source:	SA2

To:	CT1

Cc:	SA1

Contact Person:

Name:	Wanqiang Zhang

Tel. Number:

E-mail Address:	zhangwanqiang AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2200937(CR3549), S2-2200938(CR3390)

1. Overall Description:

SA2 thanks CT1 for their LS on use of network provided “Indication of country of UE location”.

In the LS, CT1 asks the use of “Indication of country of UE location” and remove the "Indication of country of UE location" from TS 23.501 and TS 23.502 if no further stage 2 requirement of that indication exist.

SA2 would like to provide the following feedback:

In the NTN WID (RP-193234) from RAN WG, UEs with GNSS capabilities are assumed. UE can be aware of the country based on its own GNSS capability.

In addition, in clause 4.23.2 of TS 24.501, a new cause value #78 "PLMN not allowed to operate at the present UE location" is defined. The UE shall store a list of "PLMNs not allowed to operate at the present UE location" and shall not attempt to access a PLMN via satellite NG-RAN access technology which is part of the list. UE selects PLMN with taking the cause value into consideration.

In addition, SA1 LS response (S1-214208/C1-217122/S2-2108994) clarified that there is no impact of “indication of country of UE location” on PLMN selection.

Based on the above analysis, the “indication of country of UE location” is useless. SA2 removes the “indication of country of UE location” from TS 23.501 and TS 23.502.

2. Actions:

To CT1 group.

ACTION: 	SA2 asks CT1 group to take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

See https://portal.3gpp.org/Home.aspx?tbid=385&SubTB=385#/