3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200960

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Key Issue: Support of long eDRX cycles in RRC_Inactive

Document for:	Approval

Agenda Item:	9.8

Work Item / Release:	FS_RedCAP_Ph2 / Rel-18

Abstract: Proposal for the new key issue for support of long DRX cycles in RRC_INACTIVE, including handling of MT signalling and data.

# 1. Introduction/Discussion

In FS_REDCAP_Ph2, the study of the MT signalling and data handling when UE is not reachable due to long eDRX cycle in RRC_INACTIVE state is proposed.

For a NR UE, the RRC_Inactive state is transparent to the CN. When there is MT data arriving in the UPF, UPF forwards the DL data to the NG-RAN. If the UE is in RRC_Inactive state, the NG-RAN shall initiate the RAN-based paging to move the UE into RRC_CONNECTED for DL data delivery.

However, in Rel-18, the eDRX cycle for RRC_Inactive state is extended to longer than 10.24s, leading to MT signalling and data handling issues.

Therefore, how to determine whether a UE is reachable and when and how handle the MT data and signalling when UE is not reachable due to long eDRX cycle in RRC_INACTIVE state need further study.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-68.

# * * * * First change (All new text) * * * *

## 5.X Key Issue#X: Support of long eDRX cycles in RRC_Inactive

### 5.X.1 General Description

This key issue is to study support of eDRX in RRC_INACTIVE state with a cycle longer than 10.24s and the handling of MT signaling and data handling when UE is not reachable due to long eDRX cycle in RRC_INACTIVE state.

This key issue will address the following aspects:

-	How the long DRX parameters (e.g. cycle length, PTW) are supported in RAN and determined.

-	When UE is unreachable due to long eDRX cycle in RRC_INACTIVE state, which entity should be able to do the UE reachability check, how to trigger the RAN-based paging for MT data delivery and where and how to buffer MT data and signaling?

# * * * * End of changes * * * *