3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200965

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Key Issue: Support of authorisation, policy and parameter provisioning for ranging services

Document for:	Approval

Agenda Item:	9.20

Work Item / Release:	FS_Ranging_SL / Rel-18

Abstract: The SL ranging is an added capability on the existing SL capability. Just like the authorisation and provision for ProSe and V2X communication, in order to perform SL ranging, we need to study what parameters should be provided and how to provide them to UE.

# 1. Introduction

In SP-211647 for SA2 FS_Ranging_SL, 5GC will make enhancements to support Ranging-based services and sidelink positioning for commercial, V2X and public safety use cases in-coverage, in partial coverage, and out-of-coverage of the 5G network.

Figure 6.37.1-1 in TS 22.261: illustration of ranging between UEs with or without 5G coverage

When a SL ranging capability UE wants to participate in SL ranging, it needs to get the necessary configuration parameters. If the parameters are preconfigured in the ME during manufacture, the UE can work without provision of configuration parameters from the network; otherwise, the UE should access to the network first to get the configuration parameters. It should be studied whether a UE is authorised for SL ranging service, what parameters should be provided, and how to provide them to UE. This is corresponding to the Work Task 1.1:

- Work Task 1: Architecture enhancements to enable Ranging Service and sidelink positioning, which include:

- Work Task 1.1: authorization and policy/parameter provisioning for a UE or a group of UEs.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-86.

# * * * * First change (all new) * * * *

## 5.x	Key Issue #X: Support of authorisation, policy and parameter provisioning

### 5.x.1	Description

A SL ranging capability UE needs necessary configuration data to perform ranging operations as well as authorisation for the SL ranging service.

RAN

UE

5GC

1. UE accesses to NW

2. Ranging service configuration parameters

Figure 5.x.1-1: ranging service authorisation, policy and parameter provisioning

In order to enable ranging service authorisation and policy/parameter provisioning, the following aspects need to be studied:

-	Identify the necessary information for ranging service authorisation and provisioning based on what is specified in TS 23.287 clause 5.1.2.1, and in TS 23.304[x] clauses 5.1.1, 5.1.2.1, and 5.1.3.1.

-	For the procedures related to range service authorisation and policy/parameter provisioning to a UE, identify the necessary enhancements based on what is specified in TS 23.287[y] clause 6.2 and TS 23.304 clause 6.2.

# * * * * End of changes * * * *