3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200966

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Key Issue: Ranging and sidelink positioning service exposure

Document for:	Approval

Agenda Item:	9.20

Work Item / Release:	FS_Ranging_SL / Rel-18

Abstract: The SL ranging service is a new 5GS capability and can be used by different consumers like a UE, an AF, and the network itself, so we need to study how to exposure this service properly.

# 1. Introduction

In clause 6.37.1 of TS 22.261, SA1 gives many ranging-based services such as consumer, smart home, smart city, smart transportation, smart retail, and industry 4.0. Correspondingly, in SP-211647 for SA2 FS_Ranging_SL, this is mapped into WT#1.5:

WT#1.5: Ranging and sidelink positioning service exposure to a UE or an Application Server who has requested the service.

In different use cases, the SL ranging service requirement varies, for example, some may require long relative distance measurement with low accuracy and high latency, and some may require high accuracy and low latency with short relative distance. The ranging service consumers may also be different, which can be a UE participating in SL ranging, a (3rd) UE, an AF, or the internal network entities. These may be considered for ranging and sidelink positioning service exposure.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-86.

# * * * * First change (All new) * * * *

## 5.x	Key Issue #X: Ranging and SL positioning service exposure

### 5.x.1	Description

SL ranging service exposure is to allow the authorised UEs, AFs, or internal NFs (e.g. AMF) to use the ranging and sidelink positioning service to enable their applications and services.

RAN

UE1

5GC

AF

UE2

3rd UE

Ranging execution

Range result exposure

NF

Figure 5.x.1-1: Ranging and sidelink positioning service exposure

It is expected that the ranging and SL positioning service exposure is supported by the 5G system, and the followings should be studied:

-	The ranging and SL positioning service is exposed by 5GS and the corresponding service requirements;

-	The architecture enhancement to support ranging and SL positioning service exposure, the required NF and NF services;

-	The authorisation of the consuming UE/AF to use ranging and SL positioning service;

-	The system procedures to enable ranging and SL positioning service exposure;

-	For roaming cases, the ranging and SL positioning service exposure related to the roaming UE by the HPLMN.

# * * * * End of changes * * * *