3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200975

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	PIN architecture assumptions

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	FS_PIN / Rel-18

Abstract: Proposes architecture assumptions

# 1. Discussion

## 1.1 PIN element definitions

The PIN SA1 requirements refer PIN element, as reported below:

  - PIN Element: UE or non-3GPP device that can communicate within a PIN.

  - PIN direct connection: the connection between two PIN Elements without any 3GPP RAN or core network entity in the middle.

  - PIN Element with Gateway Capability (PEGC): a UE PIN Element that has the ability to provide connectivity to and from the 5G network for other PIN Elements.

  - PIN Element with Management Capability : A PIN Element with capability to manage the PIN.

However from SA2 point of view, we define functionalities which can be supported on different device, For instance a UE can be a PIN element, a PIN Element with Gateway Capability or Management capabilities, or any Combination of them, so it is more flexible and generic to define the functionalities related to Personal IoT Network allowing a device either a UE or a Non-3GPP device to support only 1 or more than 1 of them allowing, in case of UE to play different roles if needed.

Proposal #1: to define of Personal IoT Network functions rather then different “elements” as done by SA1

    - PIN Element: the PIN Element is a UE or Non-3GPP device supporting PIN Element Functions which enables the realization of a Personal IoT Network

    - PIN Element Function (PEF): The function enabling the PIN network communication and supporting the PIN specific procedure, e.g. PIN discovery, etc..

    - PIN Element Gateway Capability function (PEGC): the function providing the connectivity to and from the 5G network for other device supporting the PIN Elements Function.

    - PIN Element Management Capability functiony (PEMC): The function providing the capability to manage the Personal IoT Network .

    - Note: The solutions description may provide a more detailed list of functionalities supported by each of the above PIN functions

    - A PIN Element can support both PIN management capability and Gateway Capability functions

    - Editor’s note: the definitions will be revised in order to reflect the agreed conclusion

## 1.2 PIN Architecture assumptions

The SID description includes the note in the following which are proposed to be captured as working assumption

  - NOTE 3: In release 18, this study only addresses the case where PIN Elements other than PEGC and PEMC use non-3GPP access (e.g. WIFI, Bluetooth). Additionally, the study assumes to reuse procedures defined for ProSe without introducing new features to sidelink, and no change to underlying WIFI, Bluetooth and other underlying non-3GPP access standards.

  - NOTE 4: In release 18, only UE can act as PEGC and PEMC.

Proposal #2: to capture Note 3 and note 4 of PIN SID as architecture assumptions with the rewording to refer to the above definitions.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-88

# * * * * First change * * * *

# 4	Architectural assumptions and principles

Editor's note:	This clause will document any architectural assumptions and principles relevant for the whole study

The Personal Network is assumed to be composed by following functions:

- PIN Element: the PIN Element is a UE or Non-3GPP device supporting PIN Element Functions, which enables the realization of a Personal IoT Network

- PIN Element Function (PEF): The function enabling the PIN network communication and supporting the PIN specific procedure, e.g. PIN discovery, etc..

- PIN Element Gateway Capability functionality (PEGC): the function providing the connectivity to and from the 5G network for other device supporting the PIN Elements Function.

- PIN Element Management Capability functionality (PEMC): The function providing the capability to manage the Personal Network .

Note: The solutions description may provide a more detailed list of functionalities supported by each of the above PIN functions

A PIN Element can support have both PIN management capability and Gateway Capability functionalities

Editor’s note: the definitions will be revised in normative phase in order to reflect the agreed conclusion

This study only addresses the case where PIN Element Function uses non-3GPP transport technologies (e.g. WIFI, Bluetooth, 5G Prose Direct Communication) for communications.

This study assumes to reuse procedures defined for 5G Prose Direct Communication (i.e. UE-to-UE communication) without introducing new features to sidelink, and no change to underlying WIFI, Bluetooth and other underlying non-3GPP standards.

This study assumes that only UE can support PEGC function and PEMC function.

# * * * * End of changes * * * *