SA WG2 Meeting #S2-149e(e-meeting)	S2-2201015

14 - 25 February 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	CATT

Title:	New KI #X: UE Positioning without UE/User Awareness

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3 / Rel-18

Abstract of the contribution: Introduce new key issue on how to obtain UE location information with the pre-condition that the network should not notify the UE by any means during the LCS session, e.g. to support regulatory requirements.

1. Discussion

There are regulatory cases (e.g. helping with police enquires), in that UE should not be notified by any means during the LCS session. But how to support the requirement has not been considered.

For some IoT or RedCap devices, when power saving is more important than fulfilling the LCS request, how to obtain UE location without notifying the UE especially when UE is CM-IDLE is also not considered.

Therefore, this pCR is proposed to add the key issue of how to obtain UE location information with the pre-condition that the network should not notify the UE by any means during the LCS session.

2. Proposal

This paper proposes to add a new key issue as following: (all new text).

* Start of change *

5.X	Key Issue X: UE Positioning without UE/User Awareness

5.X.1	Description

There are regulatory cases (e.g. helping with police enquires), in that UE should not be notified by any means during the LCS session. But how to support the requirement has not been considered.

For some IoT or RedCap devices, when power saving is more important than fulfilling the LCS request, how to obtain UE location without notifying the UE especially when UE is in CM-IDLE or RRC_INACTIVE state is also not considered.

To support the requirements above, this key issue will study:

-	Which NF and how to decide not to notify UE/User during the LCS session, e.g. what information is considered to make the decision;

-	Functionality and procedure enhancements to guarantee not to notify UE/User during the LCS session when UE is in CM_CONNECTED, CM_IDLE or RRC_INACTIVE state;

-	What UE location information is provided to LCS Client/ AF to fulfil the LCS QoS when the UE is in CM_IDLE or RRC_INACTIVE state and if the UE is not notified during the LCS procedure.

* End of change *