SA WG2 Meeting #149E (e-meeting)	S2-2201067

February 14 – 25, 2022, Elbonia	(revision of S2-220xxxx)

Source:	Nokia, Nokia Shanghai Bell.

Title:	New key issue: KI for objective #2 – Improved support of RAs including TAs supporting Rejected S-NSSAIs

Document for:	Approval

Agenda Item:	9.14 Study on Network Slicing Phase 3

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This paper proposes KI for objective #2

# Discussion

This paper proposes Key Issue for FS_eNS_Ph3 objective #2

2.	Study whether and how to initiate a registration for a rejected S-NSSAI that was rejected in a first TA of the RA but may be available in another TA of the RA.

# Proposal

It is proposed to agree the following changes into TR 23.700-41 on FS_eNS_Ph3.

* * * * Start of 1st Change (All new text) * * * *

## 5.X	Key Issue #X: Improved support of RAs including TAs supporting Rejected S-NSSAIs

### 5.X.1	Description

In the 5GS specifications up to rel-17, a UE that registers with a S-NSSAI which is not supported in the current TA, but is supported in some of the other TAs of the RA, receives a rejected S-NSSAI indicating this S-NSSAI is not supported in the RA. The UE therefore can only attempt to request this S-NSSAI again when it camps in a cell outside the current RA, despite it could be argued in an ideal scenario it should be able to do it also in TAs inside the RA if any of the TAs in the RA support the rejected S-NSSAI. To avoid delaying the UE to be able to request this S-NSSAI when it camps in a TA that supports the S-NSSAI, the only solution available today is to restrict the RA to only include the TAs where this S-NSSAI is not supported. This however may be suboptimal from the standpoint of signalling load and power consumption.

This Key Issue will study whether and how to enable the UE to be able to Request a S-NSSAI, which was rejected when it was camping in a first TA of the RA which does not support the S-NSSAI, when it camps in a cell of a second TA where the S-NSSAI is supported, while still allowing the AMF to include this second TA in the RA.

* * * * End of Changes * * * *