SA WG2 Meeting #149e	S2-2201089

# February 14th – 25th, 2022; Elbonia               		(revision of S2-220)

Source: 	Intel

Title: 	FS_SFC architectural assumptions and Principles

Document for: 	Approval

Agenda Item: 	8.X

Work Item / Release:	FS_SFC / Rel-18

Abstract of the contribution: This PCR provides architectural assumptions and principles for FS_SFC study in Release 18.

# 1	Discussion

void

# 2 Proposal

It is proposed to update TR 23.700-18 on FS_SFC as follows:

*** BEGIN CHANGES ***

# 4	Architectural Assumptions and Principles

The concept of the SFC is as shown in Figure 4-1 and the following assumptions are applied in this study:

- Each service function (SF) is an opaque processing element of network functions in user plane.

- The initial classification for the service chain to be applied to traffic flow is performed by 5GC.

- The 3GPP OAM is responsible for the resource management of SFs, service function paths (SPF), and service function chains, which is out of scope of this study.

- Several service function chains and SFPs can be simultaneously applied within 5GC of an operator network to meet various business requirements with third parties based on service level agreements.

- Metadata/context data can be shared amongst SFs and classifiers, between SFs, and between external systems and SFs (e.g., orchestration, PCF, AF), as stablished in [RFC7665]

Figure 4-1: The concept of the service function chaining

For all objectives, the architectural assumptions and principles include:

- Solutions shall build on the 5G System architectural principles as in TS 23.501 [3], including flexibility and modularity for newly introduced functionalities.

- Service path (i.e. for visiting the service functions) is traversed over N6 after PSA UPF(s) in 5G network.

- Existing (per 3GPP R17) deployments flexibility should be preserved in deployments of SFC in 5G network.

- The high-level diagram for enabling SFC capabilities in 5GC with interaction among different planes and interaction between 5GC and data network, is as shown in Figure 4-2:

Figure 4-2: high-level diagram for enabling SFC capabilities in 5GC

*** END CHANGES ***