SA WG2 Meeting #149E (e-meeting)	S2-2201126

Elbonia, February 14 – 25, 2022,	(revision of S2-220xxxx)

Source:	Lenovo, Motorola Mobility

Title:	New KI for WT#3 – Providing the UE with information for VPLMN selection when using non-3GPP access

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This paper proposes a new KI for WT#3 considering also the use of non-3GPP access (resulting from Rel-18 FS_5WWC_Ph2 scope).

# Discussion

The FS_eNS_Ph3 WT #3 (providing VPLMN network slice info to roaming UE) states:

3.	Study whether and how to support the following stage one Rel-18 EASNS requirements related to roaming specified in TS 22.261 clause 6.1.2.1, i.e. Requirement on enhancement the information available to the UE in roaming scenarios regarding the availability of network slices in VPLMNs available in the roaming country, in order to allow the UE to select and obtain services from the VPLMN supporting the network slices which UE may wish to use.

Additionally, the FS_5WWC_Ph2 states for WT #3:

Work task #3 Trusted/untrusted Non-3GPP access network

- Void

- How to select a TNGF/N3IWF that supports the S-NSSAI(s) needed by the UE.

Apparently, the FS_5WWC_Ph2 WT #3 should be also able to support roaming cases where the TNGF/N3IWF may be in a serving VPLMN.

We need to observe both use cases:

- The current N3IWF selection involves two steps: 1) the UE selects a PLMN; and 2) the UE selects an N3IWF in the selected PLMN.

- The selection of trusted non-3GPP access network (TNAN) also includes 2 steps: 1) a PLMN selection and 2) a trusted non-3GPP access network is which provides 5G connectivity (i.e. which includes TNGF selection).

From cases A and B above, it becomes clear that the first step involves PLMN selection. Having this, the PLMN selection for non-3GPP access should be also part of the network slicing configuration provided to the UE, which is the main objective of FS_eNS_Ph3 WT #3 topic.

Further, during discussions in the FS_5WWC_Ph2 SID opinions were raised that the PLMN selection for non-3GPP access should be studied within the FS_eNS_Ph3 WT #3 topic.

Proposal: The FS_eNS_Ph3 WT #3 should also consider the selection of VPLMN for non-3GPP access.

# Proposal

It is proposed to agree the following changes to TR 23.700-41.

* * * * Start of 1st Change (All new text) * * * *

## 5.X	Key Issue #X: Support of providing VPLMN network slice information to a UE

### 5.X.1	Description

[text capturing the KI description from FS_eNS_Ph3 WT #3]

The scope of this key issue includes selection of VPLMN when using both 3GPPP access type and non-3GPP access type. Specifically for non-3GPPP access type, the solutions will address how to select VPLMN for N3IWF selection and trusted non-3GPP access network (TNAN) selection.

NOTE Y:	Coordination with the progress of FS_5WWC_Ph2 can be needed for VPLMN selection when using non-3GPP access.

* * * * End of Changes * * * *