SA WG2 Meeting #S2-149E	S2-2201132

14 February – 25 February 2022, Electronic, Elbonia	(revision of S2-220xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0094 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the MBS SAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the lat meeting, the MBS SAI idea is agreed, but the solution is not specified. This CR proposes to complete the solution for the MBS SAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR resolves the EN in clause 6.5.4, and proposes to use MBS SAI to replace MBS XXX ID. It further clarify how the SAI work in 5MBS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to use the MBS XXX ID is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.4, 9.1.2.2, 9.3.2.2, 9.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 6.5.4	MBS SAI ID

The MBS XXX ID is used for broadcast MBS session to guide the frequency selection of the UE.

NOTE 1:	The MBS XXX ID is also used to avoid broadcast the TMGI list in the SIB.

MBS XXX ID identifies a preconfigured area within which the cell(s) announces the MBS XXX ID and the associating frequency (details see TS 38.300 [9]). MBS XXX ID and their mapping to frequencies are provided to RAN nodes via OAM.

Based on this configuration, RAN nodes announce in SIBs about the MBS XXX IDs and frequencies.

MBS XXX ID and related service area (e.g. cell list, TAI or TAI list) is configured in the MB-SMF. During the MBS Session Creation, the MB-SMF determines the MBS xxx ID(s) according to the MBS service area and returns the MBS xxx ID(s) to the AF via NEF/MBSF in the response.

NOTE 2:	The pre-configured service area of MBS XXX ID may be bigger than the MBS service area. E.g. One MBS XXX ID contains two cells, but the MBS Broadcast service is only provided in one cell.

The MBS XXX ID(s) of a broadcast MBS session are communicated in the service announcement towards the UE. The UE compares those MBS XXX IDs(s) with the MBS XXX ID(s) in SIBs for frequency selection.

During the MBS session Start, the MB-SMF may include the MBS XXX ID(s) and associated MBS service area (e.g. cell list, TAI list) to AMF. The AMF select the NG-RAN node and send the MBS XXX ID(s) and associated MBS service area to NG-RAN nodes. When the NG-RAN receives the MBS XXX ID, it broadcast the configuration for this MBS service, e.g. TMGI and radio configuration in the air interface (details see TS 38.300 [9] and TS 38.331[x]).

Next CHANGES

9.1.2.2	Nmbsmf_TMGI_Allocate service operation

Service operation name: Nmbsmf_TMGI_Allocate

Description: This service is used by the NF Service Consumer to request the allocation of TMGI(s) or request to refresh previously allocated TMGI(s).

Inputs, Required: Number of TMGIs.

Inputs, Optional:  TMGI(s) (i.e. the TMGI(s) to be extended the expiry time).

Outputs, Required: TMGI(s), Expiry Time of the TMGI(s).

Outputs, Optional: MBS XXX ID list.

Next CHANGES

9.3.2.2	Namf_MBSBroadcast_ContextCreate service operation

Service operation name: Namf_MBSBroadcast_ContextCreate

Description: This service operation is used to create the broadcast session context towards the AMF.

Inputs, Required: MBS Session ID, Broadcast service area, N2 container (e.g. MBS Session ID, MBS QoS profile, Broadcast service area).

Inputs, Optional: Area Session ID, MBS XXX ID list, Optional parameters in the N2 container: MBS IP Multicast Tunnel Info, Area Session ID.

Outputs, Required: Result Indication.

Outputs, Optional: N2 container (NG-RAN MBS Tunnel Info).

9.3.2.3	Namf_MBSBroadcast_ContextUpdate service operation

Service operation name: Namf_MBSBroadcast_ContextUpdate

Description: This service operation is used to update the broadcast session context towards the AMF.

Inputs, Required: MBS Session ID.

Inputs, Optional: Broadcast service area, MBS XXX ID list, N2 container (MBS Session ID, Possible MBS QoS profile, Possible Broadcast service area, Possible Area Session ID, Possible MBS IP Multicast Tunnel Info).

Outputs, Required: Result Indication.

Outputs, Optional: N2 container (MBS Session ID, NG-RAN MBS Tunnel Info).

END OF CHANGES