SA WG2 Meeting #149e	S2-2201140

# February 14th – 25th, 2022; Elbonia

Source: 	Nokia, Nokia Shanghai Bell

Title: 	FS_eLCS_Ph3 TR KI related with WT1.1

Document for: 	Approval

Agenda Item: 	9.10

Work Item / Release:	FS_eLCS_Ph3 / Rel-18

Abstract of the contribution: FS_eLCS_Ph3 TR KI related with WT1.1

# 1	Discussion

The FS_eLCS_Ph3 SID justification reads

In those edge computing deployed scenarios, some issues (e.g. how to reduce the location services latency from E2E perspective, how to expose the UE location estimate to application in the edge) have not been studied yet. These issues need to be investigated and resolved to enhance the location service of edge computing.

Due to lack of time in Rel-17 time window, some aspects have been de-prioritized from the scope of the eLCS_ph2 (Please refer to SP-190452 for SID proposal), e.g. positioning via user plane transmission, coordination of positioning signalling transmitted via control plane path and user plane path, location continuity support. In Rel-18, those items may need to be re-considered.

The FS_eLCS_Ph3 SID objectives reads

- WT#1.1: Investigate potential architectural enhancements to location service (e.g. in scenarios like edge computing, and other), i.e. support Positioning Signalling via user plane.

# 2 Proposal

It is proposed to update TR 23.700-71 as follows

*** Start of changes ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 38.305: " Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[x1]	OMA-AD-SUPL-V2_0: "Secure User Plane Location Architecture Approved Version 2.0".

[x2]	OMA-TS-ULP-V2_0_6: "UserPlane Location Protocol Approved Version 2.0.6".

*** Next change ***

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

LMF		Location Management Function

LPP		LTE Positioning Protocol

LPPe		LPP extension

LCS		LoCation Service

*** Next change ***

# 5	Key Issues

## 5.X	Architectural Enhancement to support User Plane LCS

### 5.X.1	Introduction

The key target of this KI is to identify the LCS feature s and enhancements required to support integration of user plane LCS.

Positioning Signalling via user plane has several benefits, such as:

- It has a more efficient communication overload with a direct connection from LCS server to UE.

  - It doesn’t require gNodeB, AMF, LMF signalling processing of RRC, NG-AP and HTTP/2 protocol stacks. A single ULP handles all the transactions.

- It can have multiple choices depending on UE support. For example, both 3GPP LPP and OMA LPPe can be possible in positioning methods’ choices

To deploy user plane LCS, it involves the following considerations:

- Different use cases like emergency/non-emergency, other regulatory cases like lawful interception and MO-LR cases, etc.

- Different deployment options including centrally deployed or local deployment ,i.e, deployed the edge data network.

For user plane LCS deployment, it can have two choices:

- user plane LCS can cooperate with LMF in the central network

In this deployment, the user plane LCS communicate with UE over N6 interface* (Reference TS 38.305 [x]):

User Plane LCS

*NOTE1: With the same approach as in [x], details of the signaling interaction between an LMF and user plane LCS can be outside the scope.

- The user plane LCS can be deployed “in scenarios like edge computing” of the edge data network to provide LCS in the edge and fits into the architecture in TS 23.548.

As per [x1], the user plane LCS can provide LCS features to emergency and non-emergency cases.*

This KI is to address architectural changes allowing user plane LCS deployment, including central deployment or deployment at the edge e.g.

- Configuration and selection of PDU Sessions to be used for the communication between UE and user plane LCS

- System procedures to support MO-LR, MT-LR, regulatory-related LCS procedures involving user plane LCS with legacy LCS call flows

*** End of changes ***