SA WG2 Meeting #S2-149E	S2-2201181

14 - 25 February, 2022, E-meeting

Source:	Xiaomi

Title:	FS_eNPN_Ph2: Architectural Assumptions and Requirements

Document for:	Approval

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_ph2/Rel-18

Abstract of the contribution: This paper proposes some Architectural Assumptions to the TR for the Study on enhanced support of Non-Public Networks; Phase 2.

Discussion

The FS_eNPN_Ph2 SID in SP-211656 includes the following objectives

5.	Address new SA1 requirements (e.g. TS 22.261 requirements from PALS work) related to NPN e.g.:

a.	Support of enabling Localized Services via a local hosting NPN e.g. by configuration of the local NPN via interaction with localized service provider.

b.	Support for enabling UE to discover, select, access the local hosting NPN and the localized services via the hosting NPN with proper authorization.

-	Support for both UE configuration for discovery and selection, and user manual selection.

c.	Support for what, how and when services are accessed via a specific network e.g. via home network or via other network (e.g. local hosting NPN).

NOTE:	The TS 22.261 requirements for Providing Access to Local Services related to Multicast/Broadcast are not part of this objective.

In TS 22.261 there are requirements for PALS as:

- Both the home network and the hosting network can be a PLMN or NPN.

- Only subscribers of a public network can roam into a PLMN. Examples of interworking scenarios between network operators and application providers for localized services are indicated in Annex I.

Proposal

It is proposed to add the following scope to the TR 23.700-08 "Study on enhanced support of Non-Public Networks; Phase 2"

*** Start of changes ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

*** Next change ***

4	Architectural Assumptions and Requirements

Editor's note: This clause will list general architectural assumptions and principles for this study.

- Both the home network and the hosting network can be a PLMN or NPN.

- Only subscribers of a public network can roam into a PLMN.

*** End of changes ***