SA WG2 Meeting #S2-149E	S2-2201182

14 - 25 February, 2022, E-meeting

Source:	Xiaomi

Title:	FS_eNPN_Ph2: KI support for PALS

Document for:	Approval

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_ph2/Rel-18

Abstract of the contribution: This paper proposes a key issue on support for Providing Access to Local Services to the TR for the Study on enhanced support of Non-Public Networks; Phase 2.

Discussion

The FS_eNPN_Ph2 SID in SP-211656 includes the following objectives

5.	Address new SA1 requirements (e.g. TS 22.261 requirements from PALS work) related to NPN e.g.:

a.	Support of enabling Localized Services via a local hosting NPN e.g. by configuration of the local NPN  via interaction with localized service provider.

b.	Support for enabling UE to discover, select, access  the local hosting NPN and the localized services via the hosting NPN with proper authorization.

-	Support for both UE configuration for discovery and selection, and user manual selection.

c.	Support for what, how and when services are accessed via a specific network e.g. via home network or via other network (e.g. local hosting NPN).

NOTE:	The TS 22.261 requirements for Providing Access to Local Services related to Multicast/Broadcast are not part of this objective.

The detailed requirements are specified in clause 6.41 Providing Access to Local Services in TS 22.261.

Proposal

It is proposed to add the following scope to the TR 23.700-08 "Study on enhanced support of Non-Public Networks; Phase 2"

*** Start of changes ***

5	Key Issues

5.X	Key Issue #X: Support for Providing Access to Local Services

Providing access to local services refers to the capability to provide access to a hosting network and a set of services offered by the hosting network provider, and 3rd party service providers including other network operators and 3rd party application providers. The services may be localized (i.e. provided at specific/limited area) and may be bounded in time. The user may become aware of the available access to local services, and the process to gain and terminate access to the hosting network and local services. This process should be efficient, and convenient from a user experience standpoint.

The following aspects will be studied as:

- How to configure the Localized Services in Hosting Network to support Providing Access to Local Services (PALS)

- UE Configuration, Provisioning, Authentication and Authorization for Providing Access to Local Services (PALS)

- How to enable UE to discover, select and access the hosting network and localized service, and how to return to home network smoothly without causing overload in the home network, including support for both Localized Services in Hosting Network, and Home Operator Services.

- Whether and how to perform charging

- Whether and how to support the Regulatory Services, e.g., PWS, LI and emergency calls, etc.

*** End of changes ***