SA WG2 Meeting #S2-149E	S2-2201214

14 February – 25 February 2022, Electronic, Elbonia	(revision of S2-220xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3579 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the FQDN(s) and IP address(es) of PVS for remote provisionning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The CT4 sends LS (S2-2200200/C4-220313) to ask the question on the transfer of the Provisioning Server (PVS) for UE Onboarding. Whether such addressing information (IP addresses and/or FQDNs) refer to a single PVS server or to distinct PVS servers. And for the latter case, how the UE use them, e.g. randomly, or with a given order and criteria. This CR proposes the UE may be locally configured with PVS IP addresses and/or PVS FQDNs or both. The UE also may obtain such information from SMF during PDU session establishment for the remote provisioning. Bucause these information are used for user plane remote provisioning and user plane provisioning protocols are out of scope of 3GPP, this CR proposes how the UE uses such information is up to UE implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The SMF may provides PVS IP addresses and/or PVS FQDNs to UE. How the UE use these information is up to UE implementation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CT4 question in S2-2200220 is not resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.10.4.2, 5.30.2.10.4.3, 5.30.2.10.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.30.2.10.4.2	Onboarding configuration for the UE

In order to enable UP Remote Provisioning of SNPN credentials for a UE, UE Configuration Data for UP Remote Provisioning are either pre-configured on the UE or provided by the ONN. UE Configuration Data for UP Remote Provisioning provided by the ONN take precedence over corresponding configuration data stored in the UE.

UE Configuration Data for UP Remote Provisioning consist of PVS IP address or PVS FQDN.

If the UE does not have any PVS IP address or PVS FQDN after the establishment of the restricted PDU Session used for onboarding, the UE may construct an FQDN for PVS discovery as defined in TS 23.003 [19].

The UE Configuration Data for UP Remote Provisioning may be stored in the ME.

The UE Configuration Data for UP Remote Provisioning (i.e. PVS IP address(es) or PVS FQDN(s), or both) may be either:

-	locally configured in the SMF of ONN; or

-	provided by the DCS to the AMF of ON-SNPN as part of the authentication procedure as specified in TS 33.501 [29] and sent by the AMF in the Nsmf_PDUSession_CreateSMContext Request message to the SMF

If the PCF is used for UP Remote Provisioning, the SMF provides the UE Configuration Data to the PCF as described in clause 5.30.2.10.4.3.

The UE Configuration Data for UP Remote Provisioning may be provided to the UE during the establishment of the restricted PDU Session as part of Protocol Configuration Options (PCO) in the PDU Session Establishment Response.

NOTE:	If there are multiple PVS IP addresses and/or PVS FQDNs in the UE, how the UE use these information (e.g. order, criteria) is up to UE implementation.

Next CHANGE

5.30.2.10.4.3	User Plane Remote Provisioning of UEs when Onboarding Network is an ON-SNPN

If Onboarding Services are provided using a restricted PDU Session for User Plane Remote Provisioning of UE via User Plane, the AMF selects an SMF used for Onboarding Services using the SMF discovery and selection functionality as described in clause 6.3.2. The S-NSSAI and a DNN of the AMF Onboarding Configuration Data may be used to discover and select an SMF for Onboarding Services, Alternatively, for SMF selection, the AMF Onboarding Configuration Data may contain a configured SMF for the S-NSSAI/DNN used for Onboarding. The AMF provides Onboarding Indication to SMF via Nsmf_PDUSession_CreateSMContext request message when a PDU Session for remote provisioning of UE via User Plane is established. If PCF is used for User Plane Remote Provisioning, SMF provides Onboarding Indication to PCF when requesting an SM Policy Association. During PDU Session establishment for remote provisioning, the AMF may provide the PVS IP address(es) and/or PVS FQDN(s) to the SMF.

When a UPF is selected for Onboarding Services, the UPF selection function described in clause 6.3.3 for normal services is applied considering the DNN used for Onboarding.

The SMF or the PCF may store S-NSSAI and DNN information used for Onboarding. Onboarding Configuration Data available to PCF (for details see TS 23.503 [45]) and/or SMF may include PVS FQDN(s) and PVS IP address(es). The SMF and the PCF may use Onboarding Indication and DNN/S-NSSAI used for Onboarding to access Onboarding Configuration Data.

When the UE registered for Onboarding successfully completes the user plane remote provisioning of SNPN credentials via the Onboarding Network, then the UE should deregister from the Onboarding Network.

Initial QoS parameters used for establishing Onboarding Services are configured in the SMF when dynamic PCC is not used.

Dynamic PCC may be used for a PDU session that is established for Onboarding Services as described in TS 23.503 [45]). The SMF may provide the UE Configuration Data (i.e. PVS IP address(es) or PVS FQDN(s)) to the PCF when requesting an SM Policy Association.

The QoS Flows of a PDU Session associated with the restricted DNN shall be dedicated to Onboarding Services. The SMF may configure PDR and FAR including PVS and DNS server IP addresses for the UPF to block any traffic that is not from or to PVS and DNS server addresses.

If the UE is registered for Onboarding, the network should apply S-NSSAI and DNN in the Onboarding Configuration Data for the PDU Session Establishment request from the UE.

Next CHANGE

5.30.2.10.4.4	User Plane Remote Provisioning of UEs when Onboarding Network is a PLMN

Onboarding Services for a UE may be provided via PLMN using PDU Session for DNN(s) and S-NSSAI(s) used for onboarding. Subscription data of such a UE shall contain the DNN and S-NSSAI used for onboarding.

The AMF selects an SMF used for User Plane Remote Provisioning using the SMF discovery and selection functionality as described in clause 6.3.2, considering the DNN and S-NSSAI used for onboarding provided by the UE or the default DNN and S-NSSAI provided by UDM.

The UPF selection function described in clause 6.3.3 is applied, considering the DNN and S-NSSAI used for onboarding.

The SMF may be configured with one or more PVS FQDN and/or PVS IP address(es) per DNN and S-NSSAI used for onboarding. When the UE requests a PDU Session for DNN and S-NSSAI used for onboarding, the SMF sends the PVS FQDN and/or PVS IP address(es) associated to the DNN and S-NSSAI of the PDU Session to the UE as part of Protocol Configuration Options (PCO) in the PDU Session Establishment Response if the following conditions are met::

-	the UE subscription data contains the DNN and S-NSSAI used for onboarding; and

-	the SMF has obtained the PVS FQDN and/or PVS IP address(es) from local configuration or from the AMF.

NOTE 1:	Based on operator policy, PDR(s) and FAR(s) can be configured to restrict traffic other than provisioning traffic between PVS/DNS server(s) and UE(s).

NOTE 2:	If UE get multiple PVS IP addresses and/or PVS FQDNs, how the UE use these information (e.g. order, criteria) is up to UE implementation.

END OF CHANGES