3GPP TSG-SA WG2#149E e-meeting	S2-2201227

Elbonia, 14-25 February 2022	(was S2-210xxxx)

Source:	Qualcomm Incorporated

Title:	TR 23.700-33 New solution to support Layer-3 UE-to-UE Relay

Document for:	Discussion/Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2/Rel-18

Abstract of the contribution: Proposes solution for supporting Layer-3 UE-to-UE Relay.

# Discussion

SA2 SID for 5G ProSe phase 2 SP-211653 has the below work task on UE-to-UE Relay support

WT#1.	Support of single NR PC5 hop UE-to-UE Relay for unicast.

NOTE 1:	WT#1 should take into account the forward compatibility for supporting more than one hop in a later release.

UE-to-UE Relay was studied as part of Rel-17 5G ProSe and multiple solutions are documented in TR 23.752. Layer-3 UE-to-UE Relaying solution documented in Sol#10 of the TR 23.752 has no impacts to the 5GS and can be supported by IP layer.

Proposal 1:  Consider Sol#10 in TR 23.752 as one of the solutions for supporting Layer-3 UE-to-UE Relaying with no impacts to the 5GS.

# Proposal

It is proposed to document the text below as a solution for supporting Layer-3 UE-to-UE Relay for the related work task/key issues.

>>>>Start Changes<<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[y]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[z]	3GPP TS 32.277: "Proximity-based Services (ProSe) charging".

>>>>Next Change<<<<

## 6.x	Solution #x: 5G ProSe Layer-3 UE-to-UE Relay based on IP routing

### 6.x.1	Description

In this solution, the ProSe 5G UE-to-UE Relay operations is supported with the following principles:

-	Authorization and configuration:

-	Only the UE authorized by the service authorization configuration can act as a ProSe 5G UE-to-UE Relay. These UEs will be configured according to the service authorization and provisioning mechanism defined in TS 23.304 [x] to operate in the UE-to-UE Relay mode.

-	ProSe 5G UE-to-UE Relay discovery:

-	The ProSe 5G UE-to-UE Relay sends out a Relay Discovery message periodically, announcing its availability for serving other UEs in the area.

-	The ProSe 5G UE-to-UE Relay also supports the query and response mode for discovery. The ProSe 5G UE-to-UE Relay listens on a configured Layer-2 ID for the query, and would respond with its address and corresponding information to enable to other UE to establish a unicast connection with it. This process is similar to the unicast L2 link establishment procedure as defined in TS 23.287 [y] clause 6.3.3.1.

NOTE 1:	The Layer-2 ID used for the discovery can be specific for UE-to-UE Relay discovery, or shared with other discoveries, e.g. UE-to-Network Relay discovery.

-	ProSe 5G UE-to-UE Relay operation:

-	Any UE that wants to make use of the ProSe 5G UE-to-UE Relay needs to establish a unicast L2 link with the UE-to-UE Relay, with IP configuration. The ProSe 5G UE-to-UE Relay allocates IP address/prefix to the other UEs.

-	As part of the unicast L2 link establishment procedure, the ProSe 5G UE-to-UE Relay stores an association of the User Info of the peer UE of the unicast link (or ProSe Service provided by the peer UE) and the IP address/prefix allocated to the UE into its DNS entries. The ProSe 5G UE-to-UE Relay acts as a DNS server to other UEs.

-	When a (source) UE needs to communicate with another (target) UE or needs to discover a ProSe service via the ProSe 5G UE-to-UE Relay, it sends a DNS query for the target UE (based on Target User Info) or for the ProSe Service to the ProSe 5G UE-to-UE Relay over the unicast link, which will return the IP address/prefix of the target UE or the IP address(es)/prefix(es) of UEs which provide the ProSe Service.

If there are multiple UEs supporting the same ProSe Service, the (source) UE can select a UE(s) based on UE implementation.

-	The source UE sends the IP data or non-IP data encapsulated in IP to the target UE or to the selected UE(s) which provide(s) the ProSe Service via the unicast L2 link to UE-to-UE Relay that returned the IP address/prefix of the target UE or UE(s) which provide(s) the ProSe Service. The ProSe 5G UE-to-UE Relay acts as an IP router, and forwards the packets to the corresponding unicast L2 link towards the target UE or UE(s) which provide(s) the ProSe Service. Each of the unicast L2 link is treated as an IP interface.

-	If there are multiple ProSe 5G UE-to-UE Relays in the proximity, UE can choose either one or more ProSe 5G UE-to-UE Relays to establish the unicast L2 link based on UE implementation. For example, the UE sends a DNS query on each of the unicast L2 link to the ProSe 5G UE-to-UE Relays. Then, the source UE may choose to use the first ProSe 5G UE-to-UE Relay that returns a positive DNS query for the target UE.

NOTE 2:	The selection of the UE-to-UE Relay may be based on local configured rules on the UE, or based on other discovery solutions.

-	QoS handling:

-	When the source UE establishes the unicast L2 link with the ProSe 5G UE-to-UE Relay, it can establish corresponding PC5 QoS Flows according to procedure defined in clause 6.3.3.1 of TS 23.287 [y]. It can also modify the PC5 QoS Flows at any time using procedure defined in clause 6.3.3.4 of TS 23.287 [y].

-	Correspondingly, the ProSe 5G UE-to-UE Relay can also establish and modify the PC5 QoS Flows using the above-mentioned procedures over the unicast L2 Link with the target UE based on PC5 Packet Filter received from the source UE during the PC5 QoS flow establishment/modification procedure or destination IP address of IP packet received from the source UE for the forwarding of source UE's traffic. The ProSe 5G UE-to-UE Relay determines the PC5 QoS parameters of PC5 QoS Flows with target UE based on corresponding PC5 QoS Flows with target UE.

-	Security handling:

-	source UE and target UE can establish bearer level security with the UE-to-UE Relay for the unicast L2 Link, using procedures defined in TS 23.287 [y].

-	If end-to-end security protection is required between source UE and target UE, IPSec can be used.

NOTE 3:	The security protection of the traffic of source UE and target UE will be specified by SA WG3.

-	Charging Support:

-	ProSe 5G UE-to-UE Relay can follow the charging solution defined in TS 32.277 [z] to report the source and target UEs and corresponding traffic to the charging function.

### 6.x.2	Procedures

Figure 6.x.2-1: 5G ProSe UE-to-UE Relay operation

Figure 6.x.2-1 provides an example operation for the 5G ProSe UE-to-UE Relay operation based on standard IP operation.

### 6.x.3	Impacts on services, entities and interfaces

There is no impact to NG-RAN, as the solution is using the existing features supported in Rel-16 NR V2X design.

UEs operate with existing IP operation, and the ProSe 5G UE-to-UE Relay supports the IP router function (for IP address allocation and traffic forwarding) and the functionality of a DNS server.

>>>>End Changes<<<<