SA WG2 Meeting #S2-149E	S2-2201251

14 - 25 February, 2022, Electronic meeting

|  | 5GMRR Doc 27_06r3 LS to 3GPP CT4 on Identification of source PLMN-ID in SBA |
| --- | --- |
| Meeting Information |  |
| Meeting Name and Number | 5GMRR #25 |
| Meeting Date | 2021-11-29 |
| Meeting Location | Virtual meeting |
| Document Information |  |
| Document Author | Mihaela Ambrozie (Vodafone) |
| Additional Contributors | Marc Balon (Orange), Gery Verwimp (BICS), Edward Yau (Syniverse), Diego Tafuto (TISparkle), Kathleen Leach (T-Mobile) |
| Document Creation Date | 2021-11-26 |
| Document Status | Approval |
| Security Classification | Non-confidential |
| Document Summary |  |
| This LS to 3GPP CT4 working group is to ask for action on Identification of source PLMN-ID in SBA in roaming scenario |  |

| Version History |  |  |
| --- | --- | --- |
| Date | Version | Author / Comments |
| 2021-11-26 | 1.0 | Mihaela Ambrozie (Vodafone) |
| 2021-12-01 | 2.0 | Mihaela Ambrozie (Vodafone) |
| 2021-12-02 | 3.0 | Mihaela Ambrozie (Vodafone) |
| 2021-12-06 | 4.0 | Mihaela Ambrozie (Vodafone) |
| 2021-12-30 | 19.0 | Mihaela Ambrozie (Vodafone) |
| 2022-01-10 | 23.0 | Mihaela Ambrozie (Vodafone) |
| 2022-01-12 | 26.0 | Mihaela Ambrozie (Vodafone) |
| 2022-01-19 | 29.0 | Mihaela Ambrozie (Vodafone) |
| 2022-01-21 | 31.0 | Mihaela Ambrozie (Vodafone) |
| 2022-01-27 | final | Pieter Veenstra (NetNumber) – Chair 5GMRR |

© GSMA © 2022. The GSM Association (“Association”) makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and disclaims liability for the accuracy or completeness or timeliness of the information contained in this document. The information contained in this document may be subject to change without prior notice. This document has been classified according to the GSMA Document Confidentiality Policy. GSMA meetings are conducted in full compliance with the GSMA Antitrust Policy.

# Liaison Statement

| Liaison Statement Title: | LS to 3GPP CT4 on Identification of source PLMN-ID in SBA |
| --- | --- |

| Source Meeting Information |  |  |
| --- | --- | --- |
| Meeting Number | Meeting Date | Meeting Location |
| 5GMRR#25 | 11/29/2021 | Virtual call |
| 5GMRR#26 | 12/16/2021 | Virtual call |
| 5GMRR#27 | 01/20/2022 | Virtual call |
| Closing data 7-days email approval | 01/27/2022 | 5GMRR email list |

| Document Details |  |  |
| --- | --- | --- |
| Document Number: | Creation Date: | Document Author: |
| 5GMRR Doc 27_06r3 | 2021-11-26 | Mihaela Ambrozie (Vodafone) |
| Originating GSMA Source: | Deadline for response: | Liaison Statement Contact |
| NG/NRG/5GMRR | See below | GSMALiaisons@gsma.com |
| Security Classification | Non-confidential |  |

| Action Required by Recipient |  |
| --- | --- |
| Internal Recipients: | NRG |
| External Recipients: | To: 3GPP CT4, SA3, SA2 |

# Background

Following the LS on Identification of source PLMN-ID in SBA from CT4 (C4-210249) and a Reply-LS on Identification of source PLMN-ID in SBA from SA3 to 5GJA (S3-212392), GSMA NG 5GMRR has discussed the need to uniquely identify the source PLMN-ID in all signalling messages over the N32-f interface, given that the cSEPP serving a PLMN may provide the pSEPP with one or multiple sender PLMN- ID(s). Below are the specific requirements from a business, operational and security perspective.

Business and Operational:

- Business Intelligence Tools – data collected needs to uniquely identify the specific PLMN-ID for the traffic. Without visibility to the PLMN-ID the need to correlate N32-c to retrieve the PLMN-ID would complicate the BSS tools and create a potential source of errors if translation tables are incorrect provisioned or not updated.

- Probing Tools/ Reporting systems - The unique PLMN-ID is needed on N32-f interfaces for probing systems to avoid complexity and cost.  In the absence of visibility to the PLMN-ID, the probing systems need to retrieve the PLMN-ID from N32-c and can no longer be stateless, making them more complex and expensive. This case is applicable when the probing tools/ reporting systems is deployed at one of the intermediaries, e.g. IPX provider.

- SLA tracing – full visibility is required by PLMN-ID to track and manage E2E SLAs per contract and inter-operator charging capabilities.  Without the PLMN-ID visibility no such services can be offered by intermediates with PRINS given that the list of PLMN-IDs is negotiated via the N32-c interface and not visible in the signalling messages on the N32-f interface.

Security

- It is understood that in some cases, the cSEPP cannot be certain which one of the PLMN-IDs to include, if none is explicitly received in an incoming request from NF consumer to the cSEPP. A solution is required on how cSEPP derives the unique appropriate PLMN ID in the request sent to the pSEPP even if the NF consumer has not included the intended source PLMN ID in the request.

- The cSEPP cannot be certain which one of the PLMN-IDs to include and consequently the receiving pSEPP cannot uniquely determine whether the included PLMN-ID is to be trusted as if coming from the NF service consumer which initiated the message.

- It is understood that in some cases, the pSEPP may detect a mismatch between the PLMN-ID included in N32-f messages and the PLMN-ID(s) from the cSEPP N32-c certificate.  A solution is required on the actions of the SEPPs in order to:

  - avoid reattempts that could fall into same result or

  - proceed and, in that case to clarify how the pSEPP will choose the content of the headers sent to the NFp.

# Actions to 3GPP CT4

GSMA NG 5GMRR kindly requests 3GPP CT4 to:

- Take the above requirements into account and update the CT4 3GPP specification where the trusted PLMN ID of each corresponding request can be provided clearly, uniquely, and reliably identified over N32-f by the intermediate parties (e.g. IPX, Roaming Hubs) and the receiving SEPP.

- GSMA would like CT4 to inform GSMA with its decision and update as soon as possible.

# Actions to 3GPP SA3

GSMA NG 5GMRR kindly requests 3GPP SA3 to take the above feedback into account and consider whether SA3 specifications require updates based on the above feedback.

# Actions to 3GPP SA2

GSMA NG 5GMRR kindly requests 3GPP SA2 to provide any feedback on probing requirements that GSMA NG 5GMRR has.

NEXT MEETINGS:

- 5GMRR#28: 1st of February 2022

- 5GMRR#29: 22nd of February 2022

- 5GMRR#30: 8th of March 2022

- 5GMRR#31: 22nd of March 2022

- 5GMRR#32: 5th of April 2022

- 5GMRR#33: 19th of April 2022