SA WG2 Meeting #149E (e-meeting)	S2-2201367

February 14 – 25, 2022, Elbonia	(revision of S2-2200375r02)

Source:	LG Electronics

Title:	Terms related to U2U relay

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract of the contribution: This paper proposes terms related to U2U relay.

Discussion

At the end of Rel-17 ProSe study, question was raised regarding terms "Source UE" and "Target UE", i.e. term about UE using U2U Relay, so the following EN was captured in TR 23.752.

Editor's note:	Terms "Source UE" and "Target UE" are used in key issues and various solutions for UE-to-UE Relay. More accurate terminology needs to be found if needed during conclusion or normative phase.

At the beginning of Rel-17 ProSe normative phase, term about UE using U2U Relay was discussed and the following term was captured in TS 23.304v0.1.0.

U2U UE: A ProSe-enabled UE that discovers or is discovered by a ProSe-enabled UE(s) via a ProSe UE-to-UE Relay; or a ProSe-enabled UE that communicates with other ProSe-enabled UE(s) via a ProSe UE-to-UE Relay.

The following term was also captured in TS 23.304v0.1.0.

ProSe UE-to-UE Relay: A ProSe-enabled UE that provides functionality to support connectivity between U2U UEs.

After that, as it was decided not to progress UE-to-UE Relay feature in Rel-17, the above terms were removed from TS 23.304.

We would like to propose to add the above terms to TR 23.700-33, with "5G ProSe" in front of the above terms.

Proposal

It is proposed to agree the following changes into TR 23.700-33.

* * * * Start of 1st Change * * * *

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and TS 23.304 [3] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] and TS 23.304 [3].

5G ProSe U2U UE: A 5G ProSe-enabled UE that discovers or is discovered by a 5G ProSe-enabled UE(s) via a 5G ProSe UE-to-UE Relay; or a 5G ProSe-enabled UE that communicates with other 5G ProSe-enabled UE(s) via a 5G ProSe UE-to-UE Relay.

5G ProSe UE-to-UE Relay: A 5G ProSe-enabled UE that provides functionality to support connectivity between 5G ProSe U2U UEs.

Editor's note:	Terms related to UEs that are involved in UE-to-UE Relay operation need to be revisited and finalized.

* * * * End of Changes * * * *