SA WG2 Meeting #149e	S2-2201560

# February 14th – 25th, 2022; Elbonia               	(revision of S2-2200670r10)

Source: 	Samsung, Huawei, ZTE

Title: 	New KI related with FS_GMEC WT3

Document for: 	Approval

Agenda Item: 	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract of the contribution: This proposes a Key Issue for FS_GMEC based on Work Task 3.

1	Discussion

The FS_GMEC SID justification reads

3GPP has specified many enablers for vertical use, for example the features defined by IIOT, NPN and URLLC. Among them, R16 5G LAN-type service offers private communication using IP and/or non-IP type communications for UE and devices behind UE, using an optimized communication path controlled by a single SMF. However, the cardinality of SMF per 5G VN is limited to one in Rel-16, even if the 5G VN is very large. Multiple SMFs should be supported, for instance for administrative domains or a large multi-site company or a large scale industrial setting that spans multiple countries. And at the same time, the SMFs need to support single/common 5G VN wide area.

In Rel-18 SA1 study on 5G Smart Energy and Infrastructure “5SEI” has concluded new requirement (clause 6.13.2, 6.28 of TS 22.261 and clause 5.2, 5.6 and 9 of TS 22.104) including that the 5G system shall allow a UE to request a communication service to send data to different groups of UEs at the same time and the 5G system shall allow different QoS policy for each group the UE communicates with.

Recently, the 5G Alliance for Connected Industries and Automation (5G-ACIA) has provided to 3GPP a whitepaper (S2-2102128) including a set of functional requirements that the 5GS has to satisfy in terms of supporting certain information exchange between 5GC and industrial application domain, and exposure of 5G capabilities. The main goal is to enable the management, operation, monitoring and use of such networks and network services from an enterprise perspective easily without having to rely on sophisticated, heavy-weight tools and in-depth knowledge on the underlying 5G technology. Some requirements regarding device management, e.g. connectivity management, connectivity monitoring, group management etc. have not yet been fulfilled and need further studies.

The FS_GMEC SID objectives read

Work Task 1: Study 5G capabilities exposure for industrial and automation applications:

-	Work Task 1.1: Enhance group attribute management and group status event reporting:

-	 set/modify the group attributes: provisioning of service area or QoS applicable to each UE of a given group

-	 subscribe to group status event reporting for the event "newly registered or (de)-registered group member"

NOTE:	The above sub-WT#1.1 assumes that existing QoS mechanism and service area mechanism are re-used for enforcement of service area or QoS applicable to each UE of a given group, thus neither new QoS nor service area enforcement mechanism will be specified.

-	Work Task 1.2: Study whether and how to enhance NEF exposure framework to enable capability exposure for  provisioning of traffic characteristics and monitoring of performance characteristics applicable to each UE of a given group

NOTE:	It is assumed that the above sub-WT#1.2 focuses on the exposure enhancements and no new enforcement mechanisms will be specified for this sub-WT. The traffic characteristics include e.g., transfer interval, data volume per cycle time, average and peak date rates, silence time interval, and PDU Session Type. The performance characteristics include communication service availability, communication service reliability, end-to-end latency, service bit rate and packet error rate. Which traffic characteristics are relevant for 5GS and which performance characteristics need to be monitored will also be studied as part of this WT.

Work Task 2: enhancements of 5G VN group communication:

-	Work Task 2.1: Support group communication for a 5G VN which supports multiple SMFs, including support of SMF redundancy for reliability of the 5G VN group communication

-	Work Task 2.2: Void

-	Work Task 2.3: Void

-	Work Task 2.4: Void

Work Task 3: Whether additional mechanism or enhancement is needed and how to support group communication allowing UE to simultaneously send data to different groups, where each group has a different QoS policy (requirement regarding 5SEI as indicated in clause 6.13.2 of TS 22.261)

Work Task 4: Void

Work Task 5: Void

Work Task 6: Void

Work Task 7: Void

Work Task 8: Void

In particular, this WT aims to study how to satisfy the following requirements from [TS 22.261, clause 6.13.2]:

-	The 5G system shall allow a UE to request a communication service to simultaneously send data to different groups of UEs at the same time.

-	The 5G system shall allow different QoS policy for each group the UE communicates with.

The original requirement is from clause 5.13 of TR 22.867 as following:

For a PMU deployed at a given bus in the grid, different types of data are generated to feed different applications of divergent criticality (such as monitoring, control, protection, state estimation, etc.). Also, the same data can be needed by different destinations requiring different QoS treatment. Because of that, these different streams of data is needed by different destinations with different QoS. Additionally, the algorithms a TSO or DSO decides to use further influences the difference in criticality (QoS requirements in other words) of various streams of data this PMU generates.

The basic scenario is summarized as that a UE with one dedicated application can belong to multiple groups, this application can generate the data and replicates multiple copies of the data, and then each copy is sent to a different destination that corresponds to a different group. The transmission of the copy within the different group needs different QoS because of the different purpose of data usage for the group.

Based on the above information, it is proposed to providing a Key Issue for FS_GMEC based on the Work Task #3.

2 Proposal

It is proposed to update TR 23.700-74 on FS_GMEC as follows.

5	Key Issues

5.X	Key Issue #X: Allowing UE to simultaneously send data to different groups with different QoS policy

5.X.1	Description

This key issue will study whether additional mechanisms or enhancement are needed to support group communication allowing a UE to simultaneously send data to different groups where each group has a different QoS policy and if yes how to.