SA WG2 Meeting #149e	S2-2200294

# February 14th – 25th, 2022; Elbonia               	    		 	 				(revision of S2-220)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3337 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handover of a PDU Session from 3GPP to untrusted non-3GPP access (I-SMF case) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2022-02-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 4.23.16.2 text is a bit misleading (additional) Step4: If an I-SMF and an I-UPF are used in the PDU Session in 3GPP access, the step 3a and 14 in clause 4.3.4.3 are performed to release the resource in the old I-SMF and old I-UPF. This step 4 (the step 3a and 14 in clause 4.3.4.3) is not taking place after step 3 of 4.23.16.2, actually the step 3a in clause 4.3.4.3 is the trigger of step 3 of clause 4.9.2.3 and Not step 3 of clause 4.9.2.2 : for the resource release clause clause 4.9.2.3 is the proper reference as there is an intermediate (I-SMF) between AMF and the (anchor) SMF; As opposed to this for the establishment over Non 3GPP access, step 3 of clause 4.9.2.2 is the proper reference as there is no I-SMf for non 3GPP access. More explanations are provided in the companion Tdoc S2-2200288 |  |  |  |  |  |  |  |  |
| Summary of change: |  | If an I-SMF and an I-UPF are used in the PDU Session in 3GPP access, the step 3a in clause 4.3.4.3 is performed to release the resource in the old I-SMF and old I-UPF and is the trigger to execute step 3 of to clause 4.9.3.2 and step14 of clause 4.3.4.3 take place after step 3 of to clause 4.9.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent and mislkeading 23.502 Clause 4.23.16.2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.16.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.23.16.2	Handover of a PDU Session procedure from 3GPP to untrusted non-3GPP access (non-roaming and roaming with local breakout)

When a PDU Session is handover from 3GPP access with an I-SMF to untrusted non-3GPP access (non-roaming and roaming with local breakout), following steps take place:

1)	step 1 of clause 4.9.2.2

2)	step 2 of clause 4.9.2.2

3)	the step 3a in clause 4.3.4.3 is performed to trigger the release the resource in the old I-SMF and old I-UPF with following addition:  the SMF indicates in the Nsmf_PDUSession_Update Request that the UE shall not be notified.

4)	Figure 4.9.2.4.2-1 step 3 involving I-SMF instead of V-SMF

5)	Figure 4.3.4.3-1 step 14 involving I-SMF instead of V-SMF

6)	Figure 4.3.4.3-1 step 16a: The SMF invokes Nsmf_PDUSession_StatusNotify (Release) indicating the release is due to a PDU Session Handover out of the I-SMF

7)	Figure 4.3.4.3-1 step 16b: the I-SMF invoke the Nsmf_PDUSession_SMContexStatusNotify (Release) indicating the Release is due to a PDU Session Handover Handover out of the I-SMF:  the AMF determines that only the SMcontext with the I-SMF is to be released but that the PDU Session is not released.

The steps 2 to 7  above shall be repeated for all PDU Sessions to be moved from 3GPP access to untrusted non-3GPP access.

NEXT CHANGE (2)

#### 4.9.2.2	Handover of a PDU Session procedure from 3GPP to untrusted non-3GPP access (non-roaming and roaming with local breakout)

Clause 4.9.2.2 specifies how to hand over a UE from a source 3GPP access to a target Untrusted non-3GPP access and how a UE can handover a PDU Session from 3GPP access to untrusted non-3GPP access. It is based on the PDU Session Establishment procedure for non-3GPP access as specified in clause 4.12.5.

Figure 4.9.2.2-1: Handover of a PDU Session from 3GPP access to untrusted non-3GPP access (non-roaming and roaming with local breakout)

1.	If the UE is not registered via untrusted non-3GPP access, the UE shall initiate Registration procedure as defined in clause 4.12.2.

2.	The UE performs PDU Session Establishment procedure with the PDU Session ID of the PDU Session to be moved as specified in clause 4.12.5.

When sending the PDU Session Establishment Accept, within the N1 SM container and in the N2 SM information, the SMF shall include all QoS information (e.g. QoS Rule(s) in N1 SM container, QFI(s) and QoS Profile(s) in N2 SM information) for the QoS Flow(s) that are applicable to the PDU Session for the target access.

3.	If the User Plane of the PDU Session is activated in 3GPP access, the SMF executes the release of resource in 3GPP access by performing step 3b, then steps 4 to 7a/7b specified in clause 4.3.4.2 (UE or network requested PDU Session Release for Non-Roaming and Roaming with Local Breakout) in order to release the resources over the source 3GPP access. Because the PDU Session shall not be released, the SMF shall not send the PDU Session Release Command to the UE. Hence, in steps 3b, 4, 6 and 7a of clause 4.3.4.2, messages do not include the N1 SM container but only the N2 Resource Release Request (resp. Ack). Since the PDU Session is not to be released, the SMF shall not execute step 11 of clause 4.3.4.2 and the SM context between the AMF and the SMF is maintained.

If the User Plane of the PDU Session is deactivated in 3GPP access, this step is skipped.

The steps 2 and 3 shall be repeated for all PDU Sessions to be moved from 3GPP access to untrusted non-3GPP access.

If the PDU Session is associated with Control Plane Only Indication, the AMF shall reject the PDU Session establishment request as the Control Plane CIoT Optimisation feature is not supported over non-3GPP accesses as described in clause 5.4.5.2.5 of TS 24.501 [25].

NEXT CHANGE (3)

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES