3GPP TSG SA WG2 Meeting #149E	S2-2200448

# Elbonia, February 14 – 25, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3507 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of VPLMN QoS constraints during mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | VPLMN applying QoS constraints based on SLA is specified for PDU Session Establishment procedure in TS23.502, however, the overall function description is missing in 23.501. This is not aligned with the general principle that TS 23.501 provides overal function description and 23.502 specifies the procedural aspects. Besides, the handling of QoS contraints at moblity, including intra 5GS mobility and EPS to 5GS mobility is not specified. During mobility procedures, it is not always possible for HPLMN to adjust the QoS during the mobility procedure and therefore target/new V-SMF may need to temporarily accept the current QoS to allow the procedure to complete, even if exceeding the SLA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description of QoS constraints at PDU Session establishment, intra 5GS mobility and EPS to 5GS mobility. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of overal function description, inconsistent specifications. Risk for rejected idle or connected mode mobility procedures during EPS to 5GS mobility or intra-5GS inter-PLMN mobility. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.1.X (new), 5.17.1.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### 5.7.1.X 	QoS aspects of home-routed roaming

In case of home-routed roaming, the V-SMF may apply VPLMN policies related with the SLA negotiated with the HPLMN or with QoS values supported by the VPLMN; such policies may result in that V-SMF does not accept the PDU Session or does not accept some of the QoS Flows requested by the H-SMF.

QoS constraints represent the QoS that VPLMN can accept based on SLA for the QoS Flow associated with the default QoS rule. The QoS constraints parameter consists of 5QI, ARP and Session-AMBR.

At PDU Session Establishment for home-routed roaming, to reduce the risk of PDU Session establishment failure due to QoS from the HPLMN not being compliant with SLA, the V-SMF provides the VPLMN local policy in QoS constraints to the H-SMF as specified in clause 4.3.2.2.2 of TS 23.502 [3].

For intra-5GS mobility with V-SMF insertion or V-SMF change (e.g., inter-PLMN mobility), as specified in clause 4.23 of TS 23.502 [3], the new/target V-SMF may validate the currently applied QoS against the QoS constraints. The new/target V-SMF provides QoS constraints to the H-SMF during the mobility procdure.  The new/target V-SMF may temporarily accept a higher QoS even if the currently applied QoS exceeds the QoS constraints. Alternatively, for the QoS parameters included in the QoS contraints, the V-SMF may locally downgrade these values before providing the corresponding QoS profiles to NG-RAN. The V-SMF may decide to release the PDU Session if the HPLMN does not provide updated QoS compliant with the QoS constraints after the mobility procedure.

**** Next Changes ****

#### 5.17.1.Y	QoS handling for home routed roaming

During mobility from EPS to 5GS, the handling of QoS constraints in V-SMF is specified in clause 4.11.1.2.2 and clause 4.11.1.3.3 of TS 23.502 [3] and follows the same principle as described in clause 5.7.1.X.

**** End of Changes ****