SA WG2 Meeting #149E (e-meeting)	S2-2200564

February 14 – 25, 2022, Elbonia	(revision of S2-220xxxx)

Source:	LG Electronics, Nokia, Nokia shanghai Bell, NEC, ZTE, Qualcomm Incorporated, Apple, Samsung, OPPO, Lenovo, Motorola Mobility

Title:	New key issue: KI for objective #3 - providing VPLMN network slice info to roaming UE

Document for:	Approval

Agenda Item:	9.14 Study on Network Slicing Phase 3

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This paper proposes KI for objective #3 - providing VPLMN network slice info to roaming UE.

Discussion

This paper proposes Key Issue for FS_eNS_Ph3 objective #3 (providing VPLMN network slice info to roaming UE).

3.	Study whether and how to support the following stage one Rel-18 EASNS requirements related to roaming specified in TS 22.261 clause 6.1.2.1, i.e. Requirement on enhancement the information available to the UE in roaming scenarios regarding the availability of network slices in VPLMNs available in the roaming country, in order to allow the UE to select and obtain services from the VPLMN supporting the network slices which UE may wish to use

Proposal

It is proposed to agree the following changes into TR 23.700-41 on FS_eNS_Ph3.

* * * * Start of 1st Change (All new text) * * * *

5.X	Key Issue #X: Support of providing VPLMN network slice information to a roaming UE

5.X.1	Description

As an outcome of SA1 EASNS (Enhanced Access to and Support of Network Slice) work, clause 6.1.2.1 in TS 22.261 [Ref#X] captures the following service requirement for a roaming UE.

For a roaming UE activating a service/application requiring a network slice not offered by the serving network but available in the area from other network(s), the HPLMN shall be able to provide the UE with prioritization information of the VPLMNs with which the UE may register for the network slice.

This key issue aims at addressing the following aspects for a roaming UE requiring a network slice not offered by the current serving network but available from other network(s):

- Study how and when the HPLMN provides the UE with information about slice availability per VPLMN and prioritization information of the VPLMNs with which the UE may register for the network slice.

- Study how and when to use the information received by the UE in the previous bullet to influence automatic PLMN selection.

NOTE 1:	For details on PLMN selection aspects and impacts on PLMN selection of solutions of this key issue, coordination with CT1 is needed.

NOTE 2:	This key issue is applicable to automatic PLMN selection only.

* * * * Start of 2nd Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[Ref#X]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

* * * * End of Changes * * * *