3GPP TSG-WG SA2 Meeting #149E e-meeting  	S2-2200586

# Elbonia, February 14 – 25, 2022	(revision of S2- 220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0507 | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove the statistics value or expected value for load level |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Table 6.3.3A-1 and Table 6.3.3A-2 are statistics type output analytics, so “the expected value” (i.e. predicted value) of the Load Level should not be included in these two tables. Correspondingly, the Table 6.3.3A-3 and Table 6.3.3A-4 are prediction type, so the “statistics value” of the Load Level should not be included in these two tables. In addition, the word “expected” should be changed to “predicted” to align with other tables. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Remove “the expected value” from Table 6.3.3A-1 and Table 6.3.3A-2. 2. Remove “the statistics value” from Table 6.3.3A-3 and Table 6.3.3A-4. 3. Update the “expected” to “predicted”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will cause confusion that the inclusion of predicted value in the statistics information or the inclusion of statistics value in the predictions information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.3.3A	Output analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the following analytics:

-	Network Slice instance load statistics information as defined in Table 6.3.3A-1.

-	Network Slice load statistics information as defined in Table 6.3.3A-2.

-	Network Slice instance load predictions information as defined in Table 6.3.3A-3.

-	Network Slice load predictions information as defined in Table 6.3.3A-4.

Table 6.3.3A-1: Network Slice instance load statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| Network Slice instances (1..max) | List of Network Slice instance(s) within the S-NSSAI. |
| > NSI ID | Identification of the Network Slice instance. |
| > Number of UE Registrations (NOTE 1) | Number of UE registrations of the Network Slice instance (average, variance). |
| > Number of PDU Sessions establishment (NOTE 1) | Number of PDU Session establishments of the Network Slice instance (average, variance). |
| > Resource usage (NOTE 1) | The usage of assigned virtual resources currently in use for the NF instances (mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [8] clause 5.7, belonging to a particular Network Slice instance. |
| > Resource usage threshold crossings (NOTE 1) | Number of times resource usage threshold is met or exceeded or crossed on the Network Slice instance provided if threshold is provided by the consumer as Analytics Filter. |
| > Resource usage threshold crossings time period (1..max) (NOTE 1) | Resource usage threshold crossing vector including time elapsed between times each threshold is met or exceeded or crossed on the Network Slice instance provided that a threshold value is provided by the consumer as Analytics Filter. |
| > Load Level (NOTE 1) | The load level of the Network Slice Instance indicated by the S-NSSAI and the associated NSI ID (if applicable) in the Analytics Filter, if Load Level Threshold is not provided by the consumer as Analytics Filter. |
| > Crossed Load Level Threshold (NOTE 1) | The Load Level Threshold that are met or exceeded by the statistics value of the Load Level if the Load Level Threshold is provided by the consumer as Analytics Filter. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

Table 6.3.3A-2: Network Slice load statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| > Number of UE Registrations (NOTE 1) | Number of UE registrations at the Network Slice (average, variance). |
| > Number of PDU sessions establishments (NOTE 1) | Number of PDU Session establishments at the Network Slice (average, variance). |
| > Load Level (NOTE 1) | The load level of the Network Slice Instance indicated by the S-NSSAI and the associated NSI ID (if applicable) in the Analytics Filter, if Load Level Threshold is not provided by the consumer as Analytics Filter. |
| > Crossed Load Level Threshold (NOTE 1) | The Load Level Threshold that are met or exceeded by the statistics value of the Load Level if the Load Level Threshold is provided by the consumer as Analytics Filter. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

Table 6.3.3A-3: Network Slice instance load predictions

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| Network Slice instances (1..max) | List of Network Slice instance(s) within the S-NSSAI. |
| > NSI ID | Identification of the Network Slice instance. |
| > Number of UE Registrations (NOTE 1) | Number of predicted UE registrations at the Network Slice instance (average, variance). |
| > Number of PDU Sessions establishment (NOTE 1) | Number of predicted PDU Session establishments of the Network Slice instance (average, variance). |
| > Resource usage (NOTE 1) | The predicted usage of assigned virtual resources for the NF instances (mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [8] clause 5.7, belonging to a particular Network Slice instance. |
| > Resource usage threshold crossings (NOTE 1) | Number of predicted times resource usage threshold is met or exceeded or crossed at the Network Slice instance provided that a threshold value is provided by the consumer as Analytics Filter. |
| > Resource usage threshold crossings time period (1..max) (NOTE 1) | Predicted Resource usage threshold vector including predicted time elapsed between times each threshold is met or exceeded or crossed on the Network Slice instance provided that a threshold value is provided by the consumer as Analytics Filter. |
| > Load Level (NOTE 1) | The load level of the Network Slice Instance indicated by the S-NSSAI and the associated NSI ID (if applicable) in the Analytics Filter, if Load Level Threshold is not provided by the consumer as Analytics Filter. |
| > Crossed Load Level Threshold (NOTE 1) | The Load Level Threshold that are met or exceeded by the predicted value of the Load Level if the Load Level Threshold is provided by the consumer as Analytics Filter. |
| > Confidence | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

Table 6.3.3A-4: Network Slice load predictions

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| > Number of UE Registrations (NOTE 1) | Predicted Number of UE registrations at the Network Slice (average, variance). |
| > Number of PDU sessions establishments (NOTE 1) | Predicted Number of PDU Session establishments at the Network Slice (average, variance). |
| > Load Level (NOTE 1) | The load level of the Network Slice Instance indicated by the S-NSSAI and the associated NSI ID (if applicable) in the Analytics Filter, if Load Level Threshold is not provided by the consumer as Analytics Filter. |
| > Crossed Load Level Threshold (NOTE 1) | The Load Level Threshold that are met or exceeded by the predicted value of the Load Level if the Load Level Threshold is provided by the consumer as Analytics Filter. |
| > Confidence | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

NOTE:	If no NSI ID is provided as Analytics Filter, slice load level related output analytics are provided according to Tables 6.3.3A-2 and 6.3.3A-4. Otherwise slice instance load level related output analytics are provided according to Tables 6.3.3A-1 and 6.3.3A-3.

# * * * * End of changes * * * *