SA WG2 Meeting #149E (e-meeting)	S2-2200992rev1

February 14 – 25, 2022, Elbonia	(revision of S2-220xxxx)

Source:	Nokia, Nokia Shanghai Bell.

Title:	New key issue: KI for objective #4 –Improved Network control of the UE behaviour control

Document for:	Approval

Agenda Item:	9.14 Study on Network Slicing Phase 3

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This paper proposes KI for objective #4

# Discussion

This paper proposes Key Issue for FS_eNS_Ph3 objective #4.

4.	Study whether and how to enhance the system to ensure network controlled behaviour of network slice usage including UE registration and PDU Session establishment (e.g. so that when performing NSAC the network slice can serve UEs/PDU Sessions with actual activity).

# Proposal

It is proposed to agree the following changes into TR 23.700-41 on FS_eNS_Ph3.

* * * * Start of 1st Change (All new text) * * * *

## 5.X	Key Issue #X: Improved network control of the UE behaviour

### 5.X.1	Description

In the 5GS specifications up to rel-17, a UE Registers/Deregisters with a Network Slice and establishes/tears down PDU sessions based on own policy taking into account network provided information such as the URSPs. However, this does not allow an operator e.g. to enforce that the UE only registers with a S-NSSAI when it is actually needed because an application is actively using it. A UE may in fact choose to register with all the Configured NSSAIs and then use the URSP just to decide with DNNs to connect to at run time. Also, it is not clear whether a UE can establish connectivity with a DNN based on own logic and URSPs at any time even ahead of usage.

Operators currently do not have the ability to request the UE to register with network slices based on e.g actual usage,   by configuration, etc.. depending on e.g what is best for the domain of application (e.g. to save battery usage one may just register based on configuration despite the URSPs are provisioned, or, when NSAC is applied on the number of UEs, the operators may want the UE to deregister from the slice subject to NSAC and register with it based on actual usage.)

There is currently no standardized means  for the operator to set a policy for deregistration of a network slice or tear down of a PDU session (e.g. the operator cannot control the time when a PDU session is released after it is last used by an application, nor can the operator decide when to deregister a UE from a network slice after there are no more PDU sessions established over it).

There is also no way for the serving PLMN to steer a UE to a preferred slice of the serving PLMN (i.e. the HPLMN or VPLMN) even if the UE may have the related HPLMN slice included in the possible connectivity options (URSP) for one application.

NOTE:	for the purpose of this Key Issue, usage of a PDU session means there is at least one application actually having an open socket that uses the connectivity of the PDU session.

This Key Issue will study how the network can control the UE and ensure the proper utilization of Slices in the system taking into account e.g. the above aspects.

* * * * End of Changes * * * *