SA WG2 Meeting #149e	S2-2200101r01

# February 14th – 25th, 2022, Elbonia

Source: 	China Mobile

Title: 	Key Issue: support UE access EHE in a VPLMN in roaming scenario

Document for: 	Approval

Agenda Item: 	9.11

Work Item / Release:	FS_ EDGE_Ph2 / Rel-18

Abstract of the contribution: Propose the key issue which is related with WT#1

# 1	Discussion

This paper is to propose a new key issue according to the WT#1 of FS_ EDGE_Ph2.

WT#1:Improvements to roaming, to support access to EHE in a VPLMN

# 2 Proposal

It is proposed to update TR 23.700-48 on FS_ EDGE_Ph2 as follows.

FIRST CHANGE

## 5.X 	Key Issue #X: Support UE access EHE in a VPLMN in roaming scenario

### 5.X.1	Description

The purpose of this key issue is to define 5GS improvements to support the UE’s access to the EHE in a VPLMN. Solutions may rely on the establishment of an LBO PDU Session or a PDU Session that includes a PSA in the HPLMN.

When solutions rely on LBO PDU sessions, they should show how the UE is aware of what DNN / S-NSSAI combination is used to establish the LBO PDU Session and access the EHE in the VPLMN.

When solutions rely on a PDU Sessions that includes a PSA in the HPLMN they need to address the following:

- 	How to authorize the PDU session to support of local traffic routing to access the EHE in the VPLMN;

- 	How to support charging for an the PDU session that supports local traffic routing to access the EHE in the VPLMN;

NOTE: 	Interaction with SA5 would be needed for this aspect

- 	Any impact on Policy and QoS control.

- 	How to configure the VPLMN ECS address to UE in roaming scenarios.

NOTE: 	In Rel-17, the ECS address is provided by UDM, so it is impossible for the UDM (in HPLMN) having the knowledge of ECS in a VPLMN. This may need some coordination with SA6.

### 5.X.2	Scenarios

void

End of CHANGE