SA WG2 Meeting #149e	S2-2200101r04

# February 14th – 25th, 2022, Elbonia

Source: 	China Mobile

Title: 	Key Issue: support UE access EHE in a VPLMN in roaming scenario

Document for: 	Approval

Agenda Item: 	9.11

Work Item / Release:	FS_ EDGE_Ph2 / Rel-18

Abstract of the contribution: Propose the key issue which is related with WT#1

# 1	Discussion

This paper is to propose a new key issue according to the WT#1 of FS_ EDGE_Ph2.

WT#1:Improvements to roaming, to support access to EHE in a VPLMN

# 2 Proposal

It is proposed to update TR 23.700-48 on FS_ EDGE_Ph2 as follows.

FIRST CHANGE

## 5.1	KI#1: Accessing EHE in a VPLMN when roaming

### 5.1.1	Description

Editor's note:	This key issue corresponds to Work Task #1 in SP-211638. This sub-clause can be further improved based on contributions.

The purpose of this key issue is to define 5GS improvements to support the UE access to the EHE in a VPLMN. Solutions may address either or both scenarios via the establishment of an LBO PDU Session or via a PDU Session that includes a PSA in the HPLMN.

Solutions relying on LBO PDU sessions should address the following:

-	how to establish the LBO PDU Session towards the correct S-NSSAI/DNN pair in order to access the EHE in the VPLMN.

Solutions relying on a PDU Sessions with a PSA in the HPLMN should address the following:

- 	how to authorize the PDU session to support of local traffic routing to access the EHE in the VPLMN;

- 	whether and how to support charging for the local traffic of a PDU session that supports local traffic routing to access the EHE in the VPLMN;

NOTE 1: 	Interaction with SA5 is expected regarding charging aspects.

NOTE 2: The latency aspect should be considered and addressed.

- 	potential impact on Policy and QoS control.

- 	how to configure the VPLMN ECS address to UE in roaming scenarios.

NOTE 3: 	In Rel-17, the ECS address is provided by UDM, so it is impossible for the UDM (in HPLMN) having the knowledge of ECS in a VPLMN. This may need some coordination with SA6.

### 5.2.2	Scenarios

Editor's note:	This clause will document the scenarios (and potential associated use cases) applicable to KI#1, if any. This clause will be removed if left empty.

For a roaming UE, accessing to EHE in VPLMN might be needed to fulfil use cases requiring edge computing. Two main scenarios should be considered:

1) UE accessing V-EHE via an LBO PDU Session.

The scenario assumes LBO PDU Session is used to accessing EHE in VPLMN for EC applications. To support other non-EC applications that needs to access their servers via HPLMN, an additional HR PDU Session is needed besides the LBO PDU Session. Hence at least two different URSP rules with different (DNN+S-NSSAI) sets are needed to trigger the two PDU Sessions. However, if the UE is in its HPLMN, the two (DNN+S-NSSAI) sets and PDU Sessions are not necessary since the EC/non-EC application can share a single PDU Session and perform selective local traffic routing via ULCL/BP.

If a UE needs to access to the EHE in VPLMN via an LBO PDU Session, how to trigger the UE to establish the LBO PDU Session is to be studied.

2) UE accessing V-EHE via a HR PDU Session.

This scenario assumes UE established a HR PDU Session during roaming. For some applications need to be local routed, the UE access to EHE in a VPLMN via the HR PDU Session. With this, a single PDU Session and (DNN+S-NSSAI) is enough to support both EC and non-EC applications in either roaming and non-roaming cases. It reduces the complexities of both network planning and UE.

To support such PDU Session, it needs to be studied that how can the UE access V-EHE via a HR PDU Session. Two sub-scenarios need to be considered as following:

2.1) HPLMN has the knowledge of EAS deployment information in VPLMN for specific services. The HPLMN triggers EAS discovery and local traffic routing in VPLMN.

2.2) HPLMN does not have the knowledge of EAS deployment information in VPLMN. The VPLMN triggers EAS discovery and local traffic routing in VPLMN.

### 5.1.3	Assumptions

Editor's note: This clause will document assumptions applicable to KI#1, if any. This clause will be removed if left empty.

End of CHANGE