3GPP TSG-SA WG2 Meeting #149E	S2-2200245r03

# 14 - 25 February, 2022, Electronic meeting     						 (revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3474 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Authentication and Subscription information checking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, KT Corp., LG Uplus, SK Telecom, Ericsson, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MINT |  |  |  |  |  | Date: |  |  | 2022-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA2 asked the feedback on the following options to SA3. During the authentication and subscription information checking, - Option 1: “Disaster Roaming indication” is transferred from AMF to AUSF/UDM - Option 2: The configuration via OAM based on operator policy and the request by the government agencies when the disaster roaming occurs (No additional signaling) Based on SA3 reply LS (S2-2200066), the corresponding editor’s notes needs to be resolved. From SA3 reply LS (S2-2200066) SA3 has discussed the security aspect on the inclusion of the Disaster Roaming indication and concluded there is no additional security issue caused by the indication. SA3 believes the indication is not used in AKA procedure. Thus, SA3 would like to request SA2 to make their own decision on whether to support the indication or not based on the use cases, and keep SA3 informed on the progress. SA3 will then update the specification under SA3 control accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to specify both options for operator’s flexible deployment and preference, by removing the current editor’s note. (Closing the SA3 related issues) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The authentication of UE and subscription information checking cannot be performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.40.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR ... CR ... TS23.502 CR#3330 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.40.4	Registration for Disaster Roaming service

For a UE to receive Disaster Roaming service from a PLMN providing Disaster Roaming service, the UE sends a NAS Registration Request message with Registration Type value "Disaster Roaming":

-	When the AMF in the PLMN providing Disaster Roaming service receives a NAS Registration Request with Registration Type set to "Disaster Roaming";

-	the AMF controls if the UE is allowed to access Disaster Roaming service in the area with Disaster Condition as specified in TS 23.502 [3] clause 4.2.2.2.2;

-	the AMF may provide the Disaster Roaming service indication to AUSF and UDM as specified in TS 23.502 [3] clause 4.2.2.2.2, and TS 33.501 [29].

NOTE 1:	The AUSF and the UDM are configured with Disaster Condition via OAM based on operator policy and the request by the government agencies. Based on this local configuration, the AUSF can execute authentication of the UE, and the UDM can provides the subscription data for a Disaster Roaming service to the AMF.

To support the Disaster Roaming service, the PLMN providing Disaster Roaming service is configured to support communication with the network entities in the HPLMN of the UE, i.e. configurations related to roaming interfaces for communication between serving PLMN and HPLMN shall be deployed in the affected entities. This communication between the PLMNs need only be enabled during the Disaster Condition.

Editor's note:	It is FFS if Disaster Roaming indication needs to be provided from AMF to SMF, and further in charging related information. This will be evaluated based on the feedback from SA WG5.

The Disaster Roaming service is limited to the impacted geographic area with Disaster Condition. The NG-RAN nodes and AMF in the PLMN providing Disaster Roaming service are configured with the area information, i.e. a list of TAIs which can be formulated by the PLMN providing the Disaster Roaming service based on the geographic area with Disaster Condition in the other PLMN(s).

The AMF in the PLMN providing Disaster Roaming service provides the mobility restriction list to the NG-RAN as specified in clause 5.3.4.1.1 considering the area with Disaster Condition, and also indicating that EPC is not an allowed core network.

NOTE 2:	From the perspective of emergency services, a UE is following existing procedures when registered for Disaster Roaming service.

* * * * End of Changes * * * *