3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200387rev2

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Key Issue: Supporting AR Communication

Document for:	Approval

Agenda Item:	9.17

Work Item / Release:	FS_NG_RTC / Rel-18

Abstract: New key Issue on IMS based AR telephony communication was proposed

# 1. Introduction

The FS_NG_RTC SID includes the following objective:

Study whether and how supporting the requirements from SA1 R18 eMMTEL WID impacts IMS architecture and functionalities:

WT#3.1: To support AR telephony communication.

NOTE 3: Coordination with SA4 (for WT#3.1) will be required.

# 2. Discussion

SA1 already finished their Rel-18 study on evolution of IMS multimedia telephony service, i.e. FS_MMTELin5G, the following requirements have been documented in TS 22.261:

  - The IMS multimedia telephony service shall support AR media processing.

SA4 already finished their Rel-17 study on evolution of AR communication, i.e. FS_5GSTAR, the conclusion has been formed in TR 26.998.

In section 4.2.2 of TR 26.998, it defines three types of device in terms of terminal capabilities (STAR/EDGAR/WLAR), while EDGAR UE has limited media rendering capability, and partial media rendering will move to the network. So IMS network media rendering is necessary in AR telephony communication for EDGAR UE.

“Compared with the instantiation for MTSI-based architecture extension, this instantiation emphasises that the IMS-AGW/MRF may support immersive media processing. It is necessary for 5G EDGAR UEs with poor media capabilities.”

But there are still some issues not finished in Rel-17 study, including:

1. The requirement of AR conversation between STAR/EDGAR/WLAR UE for IMS network is not precisely defined.

2. The impact on IMS architecture to support AR conversation signalling and media processing needs further analysis.

# 3. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-87.

# * * * * First change * * * *

## 5.X	Key Issue #X: IMS based AR telephony communication

### 5.X.1	Description

The Key Issue is to study the IMS architecture and solution for AR telephony communication including:

-

Study, and identify IMS network enhancements required to support AR telephony communication including signalling and media processing for different types of AR-capable UEs.

NOTE 1:	Coordination with SA4 will be required, the AR media and AR specific data format, codecs and media processing procedures need to be defined in SA4.

# * * * * End of changes * * * *