3GPP TSG-SA WG2 Meeting #149E	S2-2200401r07

# E-Meeting, 14 – 25 Feb. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0707 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eNPN: URSP provisioning when UE accesses an SNPN using CH credentials |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2022-01-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - As described in Disc. paper in S2-2200400, current URSP provisioning does not consider the scenario when UE accesses an SNPN using CH credentials (i.e., Use cases (c), (d), (e) in the above reference). This scenario and related handling depending on the use case should be specified. - Furthermore, in Stage 3 (clause 4.2.2.2 of TS 24.526), it is specified that: The HPLMN may pre-configure the UE with URSP in the ME or in the USIM and the SNPN(s) may pre-configure the UE with URSP in the corresponding entry of the "list of subscriber data" stored in ME, or the HPLMN and the SNPN(s) may provide URSP to the UE by signalling as described in annex D of 3GPP TS 24.501 [11]. However, URSP provisioning in TS 23.503 is missing the (subscribed) SNPN part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the description of the URSP provisioning when UE accesses an SNPN using CH credentials - Add the description of URSP provisioning by SNPN(s) to align with Stage 3 specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Unspecified handling of URSP rules when accessing an SNPN using CH credentials i.e. whether and how URSP rules are indicated to the UE and how the UE can determine what URSP rules to apply in this registered SNPN. This can render SNPN access using CH credentials inapplicable and/or make UE behavior inconsistent across misc. implementations. - Conflicting requirement with Stage 3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** FIRST CHANGE ****

#### 6.6.2.2	Configuration and Provision of URSP

The UE may be provisioned (signalled) with URSP rules by PCF of the HPLMN. When the UE is roaming, the PCF in the HPLMN may update the URSP rule in the UE. For URSP rules, the UE shall support the provisioning (signalling) from the PCF in the HPLMN, as specified in TS 24.501 [22]. In addition, the UE may be also pre-configured with URSP rules (e.g. by the operator).

An SNPN-enabled UE may be provisioned (signalled) with URSP rules by PCF of the SNPN. The PCF in the SNPN may update the URSP rules in the UE. For URSP rules, the UE shall support the provisioning (signalling) from the PCF in the SNPN as specified in TS 24.501 [22]. In addition, the UE may be also pre-configured with URSP rules (e.g. by the operator of the SNPN).

When an SNPN-enabled UE accesses an SNPN using credentials from a Credentials Holder (CH), the UE may also be provisioned (signalled) with URSP rules by the PCF the SNPN, unless the UE has been configured by the CH to not accept URSP rules signaled by an SNPN that the UE accesses using CH credentials.

In case of a UE accessing an SNPN using CH credentials, the URSP rules applicable in this SNPN are following (in precedence order):

-	the URSP rules, if any, provisioned (signalled) by PCF of the SNPN accessed using CH credentials when registered in this SNPN;

-	the URSP rules, if any, provisioned (signalled) by PCF of the network (HPLMN or SNPN) holding the credentials when previously registered in this network;

-	the URSP rules, if any, pre-configured in the UE.

-	provisioned (signalled) by PCF of the SNPN accessed using CH credentials when registered in this SNPN.

Only the URSP rules provisioned (signalled) by the PCF are used by the UE, if both URSP rules provisioned (signalled) by the PCF and pre-configured URSP rules are present. If no URSP rule is provisioned (signalled) by the PCF, and the UE has pre-configured rules configured in both the USIM and ME, then only the pre-configured URSP rules configured in the USIM is used.

If the PCF receives application guidance for URSP determination that may apply to a given UE from UDR as specified in clause 4.15.6.7 of TS 23.502 [3], the PCF may verify the requested parameters (which are described in clause 4.15.6.10 of TS 23.502 [3]) with regards to the existing URSP rules and (re-)compose the URSP rules for the UE as described in clause 6.2.4 of TS 23.548 [33].

**** END OF CHANGES ****