3GPP TSG-WG SA2 Meeting #149E meeting	S2-2200459r05

Elbonia, February 14 – 25, 2022	(revision of S2-220xxxx)

Source:	Lenovo, Motorola Mobility, Broadcom, Charter Communications, CableLabs, Intel, Ericsson, Nokia, Nokia Shanghai Bell

Title:	Key issue on Support of non-3GPP accesses for SNPN services

Document for:	Approval

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_ph2/Rel-18

Abstract of the contribution: It is proposed to introduce a key issue on support of non-3GPP access for SNPN services.

# 1. Discussion

The Rel-18 FS_eNPN_ph2 SID describes the WT#2 as follows:

2.	Support for non-3GPP access for SNPN.

We need to further discuss and agree on the scope of the Non-3GPP access (N3GPP) to be studied within WT#2.

In Rel-16 it is specified that the SNPN services can be accessed via a PLMN as described in TS23.501 Annex D.3 "Support for UE capable of simultaneously connecting to an SNPN and a PLMN".  Such access to the SNPN can be described as indrirect access.

The scope of the Rel-18 FS_eNPN_ph2 SID is to study direct access to the SNPN via non-3GPP access networks.

One open question is which types of non-3GPP access networks to study.  The authors of this paper believe that is beneficial to provide a tool-box to the SNPN operator to integrate various non-3GPP access technologies (e.g. trusted or untrested WLAN, Wireline) directly connected to the 5G Core Network.

Therefore this paper proposes to study all three types of non-3GPP access technologies: untrusted N3GPP access, trusted N3GPP access and wireline 5G access.

# 2. Proposal

It is proposed to add the following new key issue to 3GPP TR 23.700-08.

* * * First Change * * * *

## 5.X	Key Issue #X: Support of Non-3GPP access for SNPN

### 5.X.1	Description

Currently the 3GPP specifications do not support direct connection to SNPN via non-3GPP access networks. There are already non-3GPP access technologies which are in use in enterprises and campuses, and it is foreseen that such non-3GPP access technologies will continue to evolve. The integration of these existing assets in the SNPN would add flexibility to the SNPN operators. Therefore, the goal is to enable the 5GS to support direct connection of non-3GPP access networks to the SNPN's 5GC and access SNPN services. Moreover, currently indirect connection to SNPN via PLMN using untrusted non-3GPP access architecture is supported as shown in TS 23.501 Annex D.3. Indirect connection to SNPN via another SNPN is not supported.

The solutions of this key issue aim to address the following scenarios:

1.	How a direct connection to SNPN via trusted non-3GPP access network (TNAN) is supported;

2.	How a direct connection to SNPN via untrusted non-3GPP access network is supported;

3.	How a direct connection to SNPN via wireline 5G Access Network is supported; and

NOTE: Co-ordination with BBF and CableLabs will take place as needed during the study.

NOTE: Support for new types of SNPN credentials for SNPN Wireline 5G Access are out of scope

4		How an indirect connection to SNPN via NG-RAN of another SNPN is supported.

5.	Whether and how SNPN onboarding and using credentials from Credentials Holders (CH) is supported when the UE accesses the SNPN via non-3GPP access.