SA WG2 Meeting #S2-149E	S2-2200491r08

14 - 25 February, 2022, E-meeting

Source:	Ericsson, Futurewei, Nokia, Nokia Shanghai Bell, Intel, InterDigital, vivo, Lenovo, Motorola Mobility, Qualcomm Inc.

Title:	Key issue (WT#5): NPN supports of providing access to localized services

Document for:	Agreement

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_ph2/Rel-18

Abstract of the contribution: This paper introduces key issues for addressing NPN related requirements of "Providing Access to Localized Services".

# Discussion

The FS_eNPN_Ph2 SID includes an objective to address new SA1 requirements related to NPN.

5.	Address new SA1 requirements (e.g. TS 22.261 requirements from PALS work) related to NPN e.g.:

The examples in the subsequent list of this objective are reflecting and categorizing the requirements of Providing Access to Localized Services which are specified in TS 22.261 clause 6.41.

a.	Support of enabling Localized Services via a local hosting NPN e.g. by configuration of the local NPN via interaction with localized service provider.

b.	Support for enabling UE to discover, select, access the local hosting NPN and the localized services via the hosting NPN with proper authorization.

-	Support for both UE configuration for discovery and selection, and user manual selection.

c.	Support for what, how and when services are accessed via a specific network e.g. via home network or via other network (e.g. local hosting NPN).

NOTE:	The TS 22.261 requirements for Providing Access to Local Services related to Multicast/Broadcast are not part of this objective.

# Proposal

It is proposed to add the following key issues to the TR 23.700-08 "Study on enhanced support of Non-Public Networks Phase 2"

*** Start of changes ***

## 5.X	Key Issue #<X>: Enabling NPN as hosting network for providing access to localized services

### 5.X.1	Description

Providing access to local services refers to the capability to provide access to a hosting network and a set of services offered by the hosting network provider, and 3rd party service providers including other network operators and 3rd party application providers. The services may be localized (i.e. provided at specific/limited area) and may be bounded in time. The user may become aware of the available access to local services, and the process to gain and terminate access to the hosting network and local services. This process should be efficient, and convenient from a user experience standpoint.

Providing access to local services creates new opportunities for users and service providers. For example, access can be provided in areas where there is no coverage provided by other networks (for example, on a fairground established far from other infrastructure), or the access and local services can be established as needed (on a short-term basis), without the need for long term business relationships, permanently installed equipment, etc.

The type of local services and access for localized services via a hosting network can be promoted and arranged through different channels. Principally the service operators (e.g., brick and mortar businesses, entertainment venues, construction contractors, first responder agencies, etc.) will provide information and proper incentive or instructions to potential users so that they will seek to access the local services via hosting networks.

The 5G network as hosting network offering access to such localized services can be either a PLMN (e.g. a PNI-NPN) or an SNPN.

Editor's note:	The definition of service operator is FFS.

It is assumed that hosting network and the localized services can be operated by different entities. Localized services may provide more than just data connectivity to end users, e.g., additional information/incentive/instructions in order to seek access to the localized services in.

Figure 5.x-1: Overview of entities for providing access to localized services

This key issue aims at addressing how to enable a NPN (i.e. a SNPN or a PNI-NPN) as a hosting network for providing access to localized services with the following aspects:

-	Define hosting NPN and identify the difference(s) between hosting NPN and NPN defined in both Rel-16 and Rel-17.

-	Define localized services and identify the difference(s) between localized services and regular services.

-	How localized service agreements (i.e. a service agreement for a localized service) for a specific occasion (time and location) are automatically established and terminated, which allow building a temporary relationship among hosting network operator and localized services operator(s) .

Editor's note:	It is FFS whether above is to be studied in the scope of SA2 and whether SA2 works on the assumption that the relationship is already available.

-	What is required to enable communication between a network operator deploying a hosting network and a localized services operator:

-	Investigate which type of interaction (e.g., configuration of the hosting network, information reporting) is needed, in such relation to enable the service operator for making the best use of the hosting network; and

NOTE: Collaboration with SA WG5 may be needed.

Editor's note:	It is FFS whether the above is in the scope of Key issue#<X> or Key issue#<Y>.

-	Investigate if any of the work above requires cooperation with other WGs, e.g. SA5, SA3 or is suited better in other WGs.

## 5.Y	Key Issue #<Y>: Enabling UE to discover/select/access NPN as hosting network and the localized services

### 5.Y.1	Description

For providing localized services to UE, the UE needs to be able to discover, select and access the hosting network for the localized services. The discovery mechanism can be based on provisioning the UE with appropriate information. TS 22.261 has defined various requirements regarding discovery, selection and access of a hosting network in 6.41.2.3, 6.41.2.4 and 6.41.2.5.

NOTE:	For the hosting network, only NPN (SNPN or PNI-NPN) is considered in this study.

The corresponding solutions need to consider following assumptions:

-	The UE can, but not necessarily, have prior subscription with the hosting network. The UE is assumed to have a subscription with the service operators of the localized services.

-	The information for discovery of hosting network offering the localized services can be provided to the UE via either hosting network or UE's home network, or UE’s serving network or localized service provider. This information allows the UE to discover, select  and access the hosting network offering the localized services.

-	Reception and use of and usage of configuration provided by a 3rd party service provider to discover and access a hosting network and localized services is subject to operator’s policy and agreement between a 3rd party service provider and operator, including the considerations of prior service agreement with a 3rd party service provider and no prior subscription to hosting network

-	If the UE is able to obtain services from two networks simultaneously, it may additionally select the hosting network

-	If the UE cannot maintain the connection to the home network while selecting the hosting network (see KI Z), the selection shall only be done on request by the user, i.e., using manual selection

-	A localized service agreement is established (see KI X)

This key issue aims at addressing the following aspects:

-	Investigate which type of information needs to be exchanged with a localized services operators so that a UE can perform discovery, selection and connection of the hosting network and access the localized services provided via the hosting network.

-	What is the provisioning mechanism and the information needed for the UE to discover, select and access suitable hosting network for localized services with possible validity conditions for accessing the hosting network offering the localized services, because of the nature of the localized service and hosting network (e.g., time and location constrains). This includes information enabling the UE to be aware of services that can be accessed via hosting NPN.

-	Discovery and selection procedures of hosting network and localized services provided via the hosting network for UE to obtain localized services. Both automatic and end user manual selection apply.

-	How a UE already registered in a serving network (PLMN or NPN) can discover a suitable hosting network and the localized services provided via the hosting network when the hosting network and/or localized services become available.

-	How to ensure the localized services is accessed by UE according to the conditions when and where the localized services are allowed to be accessed by the UE.

-	How the UE is provisioned with credentials (if required) to access the selected localised services provided via the hosting network.

-	Mechanisms to authenticate and authorize UE to access the hosting network.

NOTE:	Security aspects (such as authentication of the UE) are in scope of SA WG3.

## 5.Z	Key Issue #<Z>: Enabling access to services via a specific network

### 5.Z.1	Description

Hosting NPN provides access to localized services. But home network operator of a UE can also utilize the hosting network based on the relationship established between hosting network operator and UE’s home operator, so that it is possible to offer the UE with a subscription from home network to access home network services via the hosting network, as well as the localized services. TS 22.261 has defined the following requirements:

- The 5G system shall be able to allow the home network to steer its UE(s) to a hosting network with the consideration of the location, times, coverage of the hosting network and services offered by the home network and hosting network.

- A localized service agreement is established (see KI X)

- The 5G system shall enable the home network operator to indicate to the UE what services are preferred to be used from the home network when the UE connects to a hosting network and the requested services are available from both the hosting and the home network

- Based on localized service agreements, the hosting network shall be able to provide required connectivity and QoS for a UE simultaneously connected to the hosting network for localized services and its home network for home network services

- A UE shall be able to connect to its home network via the hosting network, if supported by the hosting network and the home network based on localized service agreements

This key issue aims at addressing the following aspects:

-	How and whether the home network, determine the service availability of a hosting network, and interacts with hosting network to authorize home network's subscribers to access home network services via the hosting network, at certain time and location, coverage of the hosting network and services offered by the hosting network.

-	How to enable UE to access both home network services and localized services via the hosting network, and seamless service continuity for home network services and localized services when UE moves between different networks. This includes how to configure UE with information enabling the UE to be aware of services that can be accessed via a specific network (e.g. home network or hosting NPN).

-	How home network determines the need to steer or instruct the UE, and how the home network steer or instruct the UE to select a hosting network for obtaining home network services or localized services or select a network for a specific service which is available from both hosting and home network.

-	Whether and how to perform charging for the use of localized services at the hosting network and provide the charging records to UEs’ home operators.

NOTE:	It is assumed that existing mechanisms can be used to support Regulatory Services, e.g., PWS and emergency services.

*** End of changes ***