3GPP TSG-SA WG2 Meeting #149E	S2-2200507

E-Meeting, 14 – 25 February 2021

Title:	[DRAFT] Reply LS on Clarification of SCG outside RA for slicing

Response to:	S2-2200026/ R3-216237 LS on Clarification of SCG outside RA for slicing

Release:	Rel-17

Work Item:	NR_Slice

Source:	SA2

To:	RAN3

Cc:	-

Contact person:	Peter Hedman

Peter (dot) hedman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:  None

1	Overall description

SA2 would like to thank RAN3 for their LS on Clarification of SCG outside RA for slicing in S2-2200026/ R3-216237.

SA2 has discussed the question quoted below and would like to provide the following answers:

RAN3 asked: if gNB can select such “other cell” outside of the RA (registration area) of the concerned UE as SCG resource (i.e. the secondary cell group in Dual Connectivity.)?

SA2 answer:

The RA is a concept used for enabling the network to be aware of where the UE is, e.g. to limit the paging of the UE.

It is assumed that the service availability within the RA is homogeneous for the UE e.g. the TAs of the RA support the network slices in Allowed NSSAI but not all the TAs of the RA necessarily support the same network slices

However, as the SCG is not used for paging nor considered when triggering Mobility Registration Updates by NAS, there is no functional issue to use a SCG cell outside the RA.

On the principle of potentially using a cell that does not support the network slice the UE is using. SA2 assumes that only cells of TAs defined to support the used network slices are allowed and that such restriction applies also to SCG. SA2 does not foresee any impacts on SA2 specifications.

2	Actions

To RAN3:

ACTION: SA2 would like to ask RAN3 to take the above into consideration.

3	Dates of next TSG CT WG1 meetings

TSG SA WG2 Meeting 150E	6 – 12 April. 2022, E-meeting

TSG SA WG2 Meeting 151E	16 – 20 May. 2022, E-meeting