3GPP TSG-SA2 Meeting # 149E (e-meeting)	S2-2200512r01

# Elbonia, 14 – 25 February, 2022										 		 	  (revision of S2-2103898)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2868 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of ATC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | LS from CT4 S2-2200054/C4-216381 requests the clarification of the use case of the Asynchronous Type Communication. As discussed in previous response to CT4 in S2-210668 that SA2 has not identified a particular use case. The exchange of LSs between SA2 and CT4 as shown in LS S2-2200054 indicates that neither SA2 nor CT4 is able to identify any use case for the ATC function. The ATC function is introduced based on the conclusion from study phase where SMF may deliver PDU session related messages that do not expect an immediate response (the NAS specification is not developed at that time). However, based on the current NAS specification, the MT signalling is always monitored by timers and time expiration will lead to the failure of the procedure. There is no particular use case can be identified that AMF can buffer a MT N1 message and deliver it later without impact to the end-to-end procedure. There is no backward compatibility issue if the function is removed since no use case is identififed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the support of Asynchronous type communication in clause 5.23. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary work in stage 3 to support a feature that has not use case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

## 5.23	Void

****************** END CHANGES ***************