3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200910r05

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, KDDI, CATT, Ericsson? Futurewei? Intel InterDigital Lenovo? Motorola Mobility? Nokia? Nokia Shanghai Bell? OPPO, Samsung? Tencent? Tencent Cloud? vivo

Title:	New Key Issue: Media unit integrated packet handling

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: Propose a new key issue to study QoS enhancement for media unit integrated treatment for WT #3.2

1. Introduction/Discussion

This contribution proposes a key issue to study the required QoS enhancement for media unit granularity treatment in 5G network as stated in WT#3.2:

WT#3.2:  Enhance QoS framework to support media units granularity (e.g., video/audio frame/tile, Application Data Unit, control information), where media units consist of PDUs that have the same QoS requirements.

Current 5G network provides QoS support in a QoS Flow level granularity. Each packet in a QoS flow are given the same QoS requirements, e.g. packet delay budget, packet error rate, etc. Within the QoS Flow, each packet is handled individually to fulfil the policy of the QoS Flow.

For XR/media services, a group of packets are used to carry payloads of specific media unit (e.g. frame, video slice).

In media layer, such a media unit are decoded/handled as a whole. For example, the frame/video slice may only be decoded in case all of the packets carrying the frame/video slice are successfully delivered. And a frame within a GOP can only be decoded by the client in case all referenced frames are successfully received. Hence the group of packets have inherent dependency on each other in media layer. Without considering such dependencies between the group of packets, 5GS may perform a scheduling with low efficiency. For example, the 5GS may randomly drop a packet but try to deliver other packets of the same frame/video slice which is useless to the client and thus waste radio resources.

Taking “frame” as an example, as shown in the figure below:

Time (ms)

Burst bitrate

Frame Interval (1/FR)

Burst length

Time (ms)

Burst bitrate

Frame Interval (1/FR)

Burst length

Figure1： Example of downlink traffic for cloud XR

The characteristic of a group of packets for a frame includes:

- Depends on the size of frame, one XR media frame are delivered via different number of packets, e.g. ~400 IP packets for one media frame of 600KB.

- The group of packets of a frame are transferred as the periodic bursts, and the interval period between two frames are 1/frame rate, e.g. the interval is 16.7ms for a video of 60FPS.

- To ensure the MTP (Motion-To-Photon) requirements, the packets group of one media frame needs to be transferred to UE within the specified time budget, e.g. 9ms in order to further media decoding and presentation pressing at UE side. To ensure correctly decoding the media and user experience, the packet loss rate for the packet group of one media frame also has requirements, e.g. 99.9% reliability for one packet group.

If such dependency between packets of a media unit (e.g. frame, video slice,) can be considered, it’s possible to enhance the efficiency and promote user experience.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# * * * * First change (all new texts) * * * *

5.x	Key Issue #x - Media Unit integrated packet handling

5.x.1	Description

Current 5G network provides QoS support in a QoS Flow level granularity. Each packet in a QoS flow is treated according to the same QoS requirements, e.g. packet delay budget, packet error rate, etc.

For XR/media services, a group of packets are used to carry payloads of specific media unit (e.g. frame, video slice/tile).

In media layer, such a media unit are decoded/handled as a whole. For example, the frame/video slice may only be decoded in case all of the packets carrying the frame/video slice are successfully delivered. And a frame within a group of packets can only be decoded by the client in case all referenced frames are successfully received. Hence the group of packets have inherent dependency on each other in media layer. Without considering such dependencies between the group of packets, 5GS may perform a scheduling with low efficiency. For example, the 5GS may randomly drop a packet but try to deliver other packets of the same frame/video slice which is useless to the client and thus waste radio resources.

Also audio samples, haptics applications or remote control operations may benefit if the 5GS considers the Media Unit characteristics.

If such dependency between packets of a media unit (e.g. frame, video slice) can be considered, it’s possible to enhance the efficiency and promote user experience.

This key issue proposes to study media unit integrated packet handling in 5G network, in which the group of packets belongs to a same media unit will be handled as a whole. The key issue includes the following aspects:

-	Which types of media unit shall be supported for media unit integrated packet handling by 5G network;

-	What information should be provided to the PCF for media unit integrated packet handling, and how to provide;

-	How the 5GS identifies that a PDU belongs to a specific media unit;

-	Whether and how to enhance the QoS model and policy control for media unit integrated packet handling;

-	Whether and how to perform the media unit integrated packet handling in RAN and/or UPF;

-	What information needs to be provided to the RAN and/or UPF to support media unit integrated packet handling over the radio interface in both the downlink and uplink direction.

NOTE: Coordination with RAN WGs may be needed.

# * * * * End of changes * * * *