3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200932r05

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, vivo, Interdigital Inc., Ericsson

Title:	KI proposal for U2U Relay

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract: Addition of a new key issue to support U2U Relay (WT#1).

# 1. Introduction/Discussion

Support of UE-to-UE Relay was studied within R17 timeframe in TR 23.752, but was not pursued in work item.

This contribution introduces the key issue for WT#1 in the SID.

WT#1.	Support of single NR PC5 hop UE-to-UE Relay for unicast.

NOTE 1:	WT#1 should take into account the forward compatibility for supporting more than one hop in a later release.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-33.

# * * * * First change (all new) * * * *

## 5.X	Key Issue #X: Support of UE-to-UE Relay

### 5.X.1	General description

This key issue intends to support single hop UE-to-UE Relay for unicast as illustrated in figure 5.X.1-1, including support for in coverage and out of coverage operation.

Figure 5.X.1-1: Example scenario of support of UE-to-UE Relay

At least the following aspects need to be studied in potential solutions:

-	How to discover a UE-to-UE Relay(s) UE in proximity.

-	How to discover Target UE’s via a UE-to-UE Relay?

-	Whether and how the network can control UE-to-UE Relay operation, at least including how to:

-	Authorize the UE-to-UE Relay, e.g. authorize a UE as UE-to-UE Relay.

-	Authorize Source/Target UEs to use a UE-to-UE Relay.

-	Provisioning policy and parameters for UE-to-UE Relay service.

-	How to establish the connection between the source UE and the target UE via UE-to-UE Relay.

-	How to provide end-to-end QoS framework to satisfy the QoS requirements (such as data rate, reliability, latency).

-	How to enhance the system architecture to provide security/privacy protection for a relayed connection.

-	How to provide a mechanism for a path changing in case of e.g. UE-to-UE Relay changes, including reducing communication disruptions and fulfilling QoS requirements.

-	How to determine whether L2 UE-to-UE Relay or L3 UE-to-UE Relay or both are supported by the source, target and relay UEs and how to make sure the Source, Target and Relay UE all use the same type of relay.

NOTE 1:	The solution should take into account the forward compatibility for supporting more than one hop in a later release.

NOTE 2:	For the involvement of NG-RAN, coordination with RAN WGs is needed.

NOTE 3:	For security/privacy protection aspects, coordination with SA WG3 is needed.

NOTE 4: This KI covers both Layer-2 and Layer-3 UE-to-UE relay cases.

# * * * * End of changes * * * *