SA WG2 Meeting #S2-149E	S2-2201000r03

14 - 25 February, 2022, Electronic meeting

Title:	LS on QoS support with Media Unit granularity

Response to:	-

Release:	Release 18

Work Item:	FS_XRM

Source:	SA2

To:	SA4

Cc:	RAN1, RAN2, RAN3

Contact Person:

Name:	Sašo Stojanovski

E-mail Address:	saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2201041rXX

1. Overall Description:

SA2 understands from TR 26.926 “Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems” that XR applications impose requirements in terms of slices, rather than in terms of single packets. SA2 understands that a slice represents a sequence of packets that includes, e.g., all the necessary information to reconstruct a video frame and is useful only if all its packets have been received by the receiver.

As currently defined in TS 23.501, the 5G QoS model is based on QoS Flows. The QoS Flow is the finest granularity of QoS differentiation in the PDU Session. A QoS Flow ID (QFI) is used to identify a QoS Flow in the 5G System. User Plane traffic with the same QFI within a PDU Session receives the same traffic forwarding treatment, but each packet in the QoS Flow is treated independently of other packets in that QoS Flow.

SA#94E has approved a Rel-18 study on architecture enhancement for XR and media services (SP-211646).

One of the work tasks in this study is defined as follows:

WT#3: Study whether and how the following QoS and policy enhancements for XR service and media service transmission are performed:

WT#3.1:  Study the traffic characteristics of media service enabling improved network resources usage and QoE.

WT#3.2:  Enhance QoS framework to support  media units granularity  (e.g., video/audio frame/tile, Application Data Unit, control information), where media units consist of PDUs that have the same QoS requirements.

WT#3.3: Support differentiated QoS handling considering different importance of media units. e.g., eligible drop packets belong to less important media units to reduce the resource wasting.

SA2 has agreed the attached definition of  Media Unit, which may represent a slice, e.g., video/audio frame/tile

.

SA2 respectfully asks SA4 to

- provide any feedback/guidance as necessary on the above mentioned definition.

- provide any input on traffic characteristics of typical media services that rely on Media Units as necessary, and

- clarify what – if any – dependency there is between the IP packets that make up a  frame/slice.

- clarify what – if any – importance/dependency between different frames/slices.

SA4’s input is intended to be documented in an Annex of TR 23.700-60.

2. Actions:

To SA4 group.

ACTION:	SA2 respectfully asks SA4 to take the above information into account and provide feedback as needed.

3. Date of Next TSG-SA WG2 Meetings:

3GPP SA2#150-e			04 - 08 April 2021			Electronic Meeting