SA WG2 Meeting #149e	S2-2201232r06

# February 14th – 25th, 2022; Elbonia               		(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell, NTT DOCOMO, Lenovo, Motorola Mobility, Huawei, HiSilicon, Vivo, CATT, Ericsson

Title: 	WT#6 Key Issue Description and Scenario

Document for: 	Approval

Agenda Item:         9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract of the contribution: Key issue description and scenario on SID WT#6

# 1	Discussion

Approved SID has this WT#6: Investigate the potential need and solutions to influence of PSA-UPF and EAS (re)location for collection of UEs, e.g. in scenarios when UE(s) should use the same EAS and are not members of a pre-defined group.

This paper proposes description and scenario corresponding to this work task.

# 2 Proposal

It is proposed to add below text to EC TR 23.700-48

# * * * * Start of changes * * * *

## 5.X	KI#4: Influencing UPF and EAS (re)location for collections of UEs

### 5.X.1	Description

Editor's note:	This key issue corresponds to Work Task #6 in SP-211638 i.e. the below work task:

Investigate the potential need and solutions to influence of PSA-UPF and EAS (re)location for collection of UEs, e.g. in scenarios when UE(s) should use the same EAS and are not members of a pre-defined group.

In particular, the key issue will study the following aspects:

-	Define a collection of UEs that should use the same EAS and/or same local part of DN and/or same DNAI and how the collection is identified.

-

-	Whether and how an AF can influence UPF and EAS (re)location for a collection of UEs

-	How to decide on a common local part of DN for the collection of UEs.

-	How to handle coordination of the UPF and EAS (re)location for collections of UEs.

-	Whether and how existing mechanisms suffice.

-	Whether and what improvements are required for EAS discovery and re-discovery for UEs belonging to a collection of UEs.

Editor's Note:	The relationship between this KI and KI#3 should be evaluated

### 5.X.2	Scenarios

Editor's note:	This clause will document the scenarios (and potential associated use cases) applicable to KI#4, if any. This clause will be removed if left empty.

There are use cases that UEs belonging to a non-predefined dynamic group should be treated the same way, and members of the dynamic group is likely to change dynamically, e.g. UE could join/leave the group randomly. For example:

There are multiple use cases and scenarios such as:

Multi-user low latency Gaming: UEs identified by application (e.g. UEs within a geographical area) need to interact with each other for accessing to the same gaming service, the UEs needs to connect to the same application server or application servers associated with the same DNAI to reduce the interacting latency from application server side.. Also, due to maintenance purposes or due to overload situation EAS would require to be relocated to another one, thus moving all registered users to the new EAS.

Platooning: In case of platooning use case, all the member UEs involved has similar attributes and requirements, for example, all UEs in a particular platoon have similar mobility characteristics i.e. moving in the same direction and with similar speed, located in proximity to each other, and so on.

The following scenarios will be investigated for this key issue:

Figure 1 Scenario#1: UEs in dynamic group connecting to the same EAS.

Figure 2 Scenario#2: UEs in dynamic group connecting to the EASs associated with the same DNAI or local part of DN

### 5.X.3	Assumptions

Editor's note:	This clause will document assumptions applicable to KI#4, if any. This clause will be removed if left empty.

# * * * * End of changes * * * *