3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2201317

Elbonia, Februray 14 – 25, 2022	(Revision of S2-2200470r02)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0081 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving ENs, cleanup and corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-02-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 7.1.2, MB-SMF updates NRF profile to remove the MBS Session info under the condition that MB-SMF performed NRF registration at MBS session creation. The condition is missing. When the multicast MBS session context is deleted from the MB-SMF, e.g. due to MBS session release, the MB-SMF updates its profile towards the NRF, i.e., removing the MBS session ID which is no longer served by the MB-SMF |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 7.1.2 Add condition “if the MB-SMF performed NRF registration at multicast MBS Session creation” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing condition and poor readability of specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START OF CHANGES ***************

### 7.1.2	MB-SMF discovery and selection for multicast/broadcast session

To facilitate the MB-SMF discovery/selection for one multicast/broadcast MBS session, the following mechanism is used:

-	The MB-SMF registers its capability related to multicast/broadcast session management (e.g. S-NSSAI(s) and the associated NSI ID(s) (if available), DNN(s), TMGI range, service area) as part of its profile to the NRF by invoking Nnrf_NFManagement_NFRegister. In addition, when an multicast session is created and the MBS session ID is not yet included in the MB-SMF profile, the MB-SMF updates its profile towards the NRF with the MB Session ID (i.e. TMGI or source specific IP multicast address).

NOTE:	The operator can preconfigure MB-SMF for specific source IP multicast address range or TMGI range.

-	When the UE joins the multicast MBS session via PDU session modification procedures, the SMF serving the PDU session invokes the Nnrf_NFDiscovery_Request Request including the MBS session ID for multicast provided by the UE and optionally other information, i.e. the S-NSSAI and the associated NSI ID (if available), DNN, etc., to query the NRF for MB-SMF information. Based on the MBS session ID and other information for query, the NRF decides whether an MB-SMF serving the MBS session exists (i.e. the NRF decides whether the requested MBS Session ID is in the profile of an MB-SMF, as defined in clause 7.1.1.2 or clause 7.1.1.3 or preconfigured by the operator).. If so, the NRF provides in Nnrf_NFDiscovery_Request Response the information of the MB-SMF currently serving the MBS session. The SMF serving the PDU session selects the MB-SMF currently serving the multicast MBS session, based on the MB-SMF information provided by the NRF. For local MBS services, the SMF takes MB-SMF service area, UE location into account when selecting the MB-SMF. If no MB-SMF serving the multicast session exists, the NRF does not provide MB-SMF profiles in the response message to the SMF as defined in TS 29.510 [19].

-	When the shared tunnel for shared delivery is established as described in clause 7.2.1.4, the AMF may invoke the Nnrf_NFDiscovery_Request Request including MBS Session ID and Area Session ID (if received from the NG-RAN), to query the NRF for the MB-SMF information. In this case, based on the MBS session ID and Area Session ID, the NRF provides in Nnrf_NFDiscovery_Request Response the MB-SMF information currently serving the MBS session and the Area Session ID.

-	When the multicast MBS session context is deleted from the MB-SMF, e.g. due to MBS session release, the MB-SMF updates its profile towards the NRF (i.e., removing the MBS session ID which is no longer served by the MB-SMF) if the MB-SMF performed NRF registration at multicast MBS Session creation.

-	During MBS session information provisioning procedures defined in clause 7.1.1.2, unless the MB-SMF information is available by other means, e.g. locally configured in the NEF/MBSF/AF, the NEF/MBSF/AF queries the NRF with information of the multicast/broadcast session (e.g. S-NSSAI(s) and the associated NSI ID(s) (if available), DNN(s)), and selects the MB-SMF(s) based on the MB-SMF information provided by the NRF. For local MBS services, the NEF/MBSF/AF takes MB-SMF service area into account when selecting the MB-SMF(s).

****************** END OF CHANGES ***************