SA WG2 Meeting #149e	S2-2201338

# February 14 – 25, 2022; Elbonia               	    (revision of S2-2200762r01)

Source: 	Lenovo, Motorola Mobility

Title: 	KI for WT#6

Document for: 	Approval

Agenda Item: 	9.3

Work Item / Release:	FS_ATSSS_Ph3 / Rel-18

Abstract of the contribution:

# Proposal

It is proposed to add the following Key Issue (KI) into TR 23.700-53, associated with the approved WT#6.

Start of Changes

# 5	Key Issues

## 5.X	Key Issue #6: Supporting MA PDU Session with one 3GPP access path via 5GC and one non-3GPP access path via ePDG/EPC

### 5.X.1	Description

This key issue considers how to support the establishment of a MA PDU Session with one 3GPP access path via 5GC and one non-3GPP access path via ePDG/EPC. A solution for this key issue complements the existing ATSSS solution that supports the establishment of a MA PDU Session with one non-3GPP access path via 5GC and one 3GPP access path via EPC (see TS 23.501 [2], clause 4.22.2.3).

The key issue addresses the following aspects:

-	How to establish a MA PDU session with a non-3GPP access path over ePDG/EPC and a 3GPP access path over 5GC.

-	How to establish an PDN Connection in EPC using the S2b procedures with untrusted non-3GPP access and designate this PDN Connection as user-plane resource associated with a MA PDU Session.

-	How to transfer a non-3GPP access path over ePDG/EPC (while keeping the 3GPP access path over 5GC) to a trusted non-3GPP access path over 5GC.

NOTE:	Handover between non-3GPP of EPC and non-3GPP access of 5GC will not be defined.

-	The impact on UE, ePDG, EPC and 5GC.

-	Which steering functionalities and which steering modes can be applied to an MA PDU Session with a non-3GPP access path over ePDG/EPC and a 3GPP access path over 5GC.

It is assumed that:

-	The UE can attach to EPC via an ePDG using the S2b procedures (see TS 23.402, clause 7.2) and can simultaneously register with 5GC over 3GPP access.

-	The study is restricted to EPC attach using the S2b procedures via untrusted non-3GPP access. EPC attach using S2c procedures via untrusted non-3GPP access are not considered.

-	Solutions are preferred that re-use (as much as possible) the existing concepts and procedures for enabling an MA PDU Session with one non-3GPP access path via 5GC and one 3GPP access path via EPC, as defined in TS 23.501 [2], clause 4.22.2.3.

End of Changes