SA WG2 Meeting #149e	S2-22001563

# February 14th – 25th, 2022; Elbonia               	(revision of S2-220115r02)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	New solution for WT2

Document for: 	Approval

Agenda Item: 	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract of the contribution: New solution for WT2

# Discussion

The GMEC SID contains

Work Task 2: enhancements of 5G VN group communication:

-	Work Task 2.1: Support group communication for a 5G VN which supports multiple SMFs, including support of SMF redundancy for reliability of the 5G VN group communication

# 2 Proposal

It is proposed to update TR 23.700-74 for FS_GMEC as follows

## 6.0	Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions |  | WT2 |  |  |
| Solution X: use of SMF sets for the reliability of the 5G VN group communications |  | X |  |  |

NEXT CHANGE (2) all text is NEW

## 6.X	Solution #X: use of SMF sets for the reliability of the 5G VN group communications.

### 6.X.1	Introduction

Void.

### 6.X.2	Functional Description

The Support of group communication for a 5G VN needs to ensure SMF redundancy for the reliability of the 5G VN group communications.

Such redundancy can be provided by the SMF set feature that has been supported by 5GC since REL-16.

Any inter SMF interactions related with intra SMF set co-ordination about a 5G VN group ( e.g. on the control of the N19 tunnel between the UPF(s) involved in a 5G VN group communication) is left outside the scope of 3GPP specifications.

NOTE: 	This solution does not assume that 5G VN group communication only happens in a local area, e.g. involving the same unique data center as a SMF Set may be deployed across multiple sites/data centers. Furthermore, to handle 5G VN groups with UE spread over a large country, I-SMF may be used to connect remote UE(s) to the (unique) SMF set serving the 5G VN group

### 6.X.3	Procedures

Following note of 23.501 [2] § 5.29.3 “PDU Session management” needs to be reworded to no more refer to a “SMF” but to a “SMF set”

NOTE 1:	The network is configured so that the same SMF is always selected for a certain 5G VN group, e.g. only one SMF registers on the NRF with the DNN/S-NSSAI used for a given 5G VN group.

Thus, the NOTE would read as follows

NOTE 1:	The network is configured so that the same SMF *set* is always selected for a certain 5G VN group, e.g. only one SMF registers on the NRF with the DNN/S-NSSAI used for a given 5G VN group.

When the UE is not located in the service area of the SMF set associated with the DNN/S-NSSAI used for a 5G VN group, per 3GPP R16 procedures, the AMF involves an I-SMF for the PDU Session.

Editor's Note:	How to support SMF set for a certain 5G VN group with/without UDSF is FFS..

Editor's Note:	Usage of I-SMF and SMF set to support 5G VN groups with UE spread over a wide area (e.g., the whole country like China) needs to be further explained.

### 6.X.4	Impacts on existing entities and interfaces

The solution impacts are on:

- Registering a SMF set (and not a SMF instance) as associated with the DNN + S-NSSAI representing a certain 5G VN group