3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2201611

Elbonia, February 14th – 25th, 2022	(revision of S2-2200964r03)

Source:	Huawei, HiSilicon

Title:	Key Issue: Ranging service procedure with the assistance of another UE

Document for:	Approval

Agenda Item:	9.20

Work Item / Release:	FS_Ranging_SL / Rel-18

Abstract: Proposes a new key issue to support ranging service procedure with the assistance of another UE. 1. Introduction/Discussion

This contribution introduces the key issue for Ranging service operation procedures with the assistance of another UE within WT#1.3 in the SID.

- Work Task 1.3: Ranging device discovery and service operation procedures between two UEs, between one UE and multiple UEs or via the assistance of another UE;

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-86.

# * * * * First change (all new) * * * *

## 5.X	Key Issue #X: Ranging service operation procedure with the assistance of another UE

### 5.X.1	General description

The ranging service operation between two UEs may need the assistance of another UE (e.g., assistant UE) if the direct ranging/sidelink positioning between the reference UE and the target UE cannot be supported. The direct communication between reference UE and assistant UE and between target UE and assistant UE are respectively supported, so that the assistant UE can help on measurement and data transmission.

This key issue addresses how to enhance the 5GS to support ranging service operation procedures with the assistance of another UE (e.g., assistant UE).

At least the following aspects need to be considered in potential solutions:

-	How to discover and determine the assistant UE for ranging service?

-	What parameters need to be exchanged between different UEs (reference UE, assistant UE, target UE) during ranging service operation procedures?

-	How the reference UE or target UE gets the ranging result with the assistance of the assistant UE during the ranging service operation procedures?

NOTE:	the direct discovery procedure between reference UE and assistant UE and between target UE and assistant UE will be developed under the key issue related to ranging direct discovery.

# * * * * End of changes * * * *